- GenRef

- v1.00

- MDOS Reference guide.

- Memory Library

- (C) Copyright 1989

J. Paul Charlton

ALL RIGHTS RESERVED

___________________________________________________________________________________

MEMORY CONTENTS               ___________________________________________________________________________________

Page

Memory management overview........................1

Calling memory functions..........................2

Available memory..................................3

Allocate memory...................................3

Release memory....................................9

Map memory........................................12

Get memory list...................................14

Declare shared memory.............................16

Release shared memory.............................18

Use shared memory.................................19

Get size of shared group..........................20

Free task.........................................21

Get memory page...................................21

Free memory page..................................22

Free memory node..................................23

Link memory node..................................23

Get memory list (system)..........................24

___________________________________________________________________________________

MEMORY MANAGEMENT OVERVIEW ___________________________________________________________________________________

The memory management routines in MDOS  are  provided  to

aid  a  programmer  in writing applications which are larger

than the 64 Kbytes directly  addressable  by  the  CPU's  16

address  lines.   They  also  serve the purpose of providing

each task with it's own private address space, separate from

memory accessible to other tasks.

Each task under MDOS can have 2 Mbytes of virtual memory,

using 21 address bits.  The 21 addresses bits consist of two

fields.  The first field includes the eight most significant

address bits, and is referred to as the virtual page number.

The second field consists of the thirteen least  significant

address bits, and is referred to as the page offset.

The physical memory in the Geneve computer has 21 address

lines,  for  a maximum of 2 MBytes of physical memory.  Like

virtual memory, each physical address can be thought of as a

21 bit address of two fields, with the first eight bit field

called the physical page number, and the final thirteen  bit

field referred to as the page offset.

NOTE:  It can be easy to confuse physical pages with  virtual

pages,  so  be  careful when reading the opcode descriptions

below.

The 16 address lines provided by the 9995  processor  can

be  thought  of  two  fields.   The  first field is the most

significant three bits of the  address  and  is  called  the

"window  number".   The  least significant thirteen bits are

the page offset.  These 16 address bits can be  referred  to

as the linear address space of the CPU.

The virtual address space and the  linear  address  space

are  of  the most interest to people writing tasks which run

under  MDOS.   The  memory   management   routines   provide

transparent  methods of assigning physical memory pages into

your task's virtual address space and transparent methods of

viewing  any  8k  block  of  a  task's virtual address space

within one of the seven usable  8k  memory  windows  in  the

linear address space.

MEM - 2                              GenREF v1.00

MDOS maintains two arrays to manage the  physical  memory

pages  belonging  to your task.  The first array, which only

contains 8 physical page numbers,  is  part  of  the  Geneve

hardware  and is called the "mapper".  The mapper is used to

assign a physical memory  page  to  each  of  the  eight  8k

windows addressable in the 9995's linear address space.  The

second array, which is actually stored as  a  singly  linked

list within MDOS, is stored in RAM under control of the MDOS

memory management  routines  and  is  referred  to  as  "the

virtual  page  list".   Each  node  in the virtual page list

consists of a physical page number  and  various  attributes

for  that page.  Various attributes used in the virtual page

list allow for pages to  be  unassigned  (correspond  to  no

useful  physical page), for pages to be shared, for pages to

be disk-resident (swapped out), and for pages to be  private

(accessible to only your task.)

___________________________________________________________________________________

CALLING MEMORY FUNCTIONS

___________________________________________________________________________________

The MDOS memory management functions must be called  from

within  a machine code program running as a task under MDOS.

You pass arguments to the memory management functions  using

only a few registers of your program's workspace.

The MDOS memory management functions are invoked  from  a

machine  code program when software trap number zero (XOP 0)

is called with a library number of 7.  The calling program's

R0  must contain the opcode of the routine within the memory

management library which is to be performed.  The  following

code fragment will allocate memory to your task.

LI        R0,1

LI        R1,7      56k bytes

LI        R2,1      starting @>2000

SETO      R3        try fast pages

XOP       @SEVEN,0

MOV       R0,R0

JNE       MEMERR

* ...

SEVEN     DATA      7

*...

GenREF v1.00                            MEM - 3

___________________________________________________________________________________

AVAILABLE MEMORY

___________________________________________________________________________________

Function                 You would use this operation in a program when you

want to determine how much memory is available for

use.  It returns the  total  number  of  8k  pages

installed,  the number of zero wait state 8k pages

available,  and  the  total  number  of  8k  pages

available (both fast and slow pages).

Parameters               R0        = 0 (opcode)

RESULTS                  R0        = 0       (no error)

R1        = total number of free pages

R2        = number of free zero wait state pages

R3        = total number of installed pages

___________________________________________________________________________________

ALLOCATE MEMORY

___________________________________________________________________________________

Function                 This routine allows you to assign  physical  pages

of  memory  from  system  list  of  free  pages to

virtual pages belonging to your  task.   You  must

use  this  function if you wish to use more memory

than your program occupied on disk  as  a  program

image  file.   You  must also use this function if

you wish to use more  than  64k  of  memory  in  a

program  you  have written.  This routine will not

reassign pages which have already been  allocated,

even  if  the  block of pages you specify overlaps

pages which have already  been  assigned  to  your

task.

On successful  return,  all  pages  in  the  range

R2..(R2+R1-1) are available for use by your task.

Parameters               R0        = 1 (opcode)

R1        = page count

R2        = starting page

R3        = speed flag 0 (use first available memory page)

<>0 (use zero wait state pages, if available)

Results                  R0        = error code

R1        = new count

R2        = fast count

MEM - 4                                 GenREF v1.00

Parameter Description

Page count          This is the number of consecutive memory pages you

wish   to   have  for  your  program,  it  is  not

necessarily the number  of  pages  which  will  be

returned  to  your  program.   As  an  example, to

allocate 20k bytes of memory for your program, you

must  actually ask for three 8k memory pages.  The

number of  pages  you  need  to  ask  for  can  be

calculated  from  the  number of bytes you need as

follows:

pages = (bytes + >1fff) / >2000

Starting page       This is the virtual page number within your task's

memory  at  which you want to allocate more memory

pages.  If you were to think of your task's memory

as   having  addresses  ranging  from  >000000  to

>1fffff (0 to 2 MB), this number  is  the  address

divided by 8192 (remainder is discarded.)

Speed flag          If this flag is non-zero,  MDOS  will  attempt  to

assign  zero wait state memory pages to your task.

If there are not  enough  zero  wait  state  pages

available  to  satisfy  your  request,  MDOS  will

assign slow pages to your task in order to satisfy

the request.

If this flag is zero, and your computer  has  512k

of  (one  wait state) RAM on the motherboard, MDOS

will first attempt to assign slow  pages  to  your

task.   If  there  are not enough slow pages, MDOS

will continue by allocating  fast  pages  to  your

task.

If this flag is zero, and your computer has  1024k

of  (zero wait state) RAM on the motherboard, MDOS

will first attempt to assign fast  pages  to  your

task.   If  there  are not enough fast pages, MDOS

will continue by allocating  slow  pages  to  your

task.

The "fast count"  returned  to  you  reflects  the

number  of fast pages allocated as a result of the

operation, and the "fast  count"  subtracted  from

the  "new  count"  returned  to  you  reflects the

number of slow pages allocated as a result of  the

operation.

If the "fast count" returned to you  is  non-zero,

and   different  from  the  number  of  pages  you

requested,  there  is  no  convenient  method   of

determining  which  pages  are fast, and which are

slow.  The easiest deterministic method of telling

which  pages are fast and which are slow is to ask

GenREF v1.00                         MEM - 5

MDOS for one page at a time, and look at the "fast

count" resulting from each single-page allocation.

Error code          0 = No error.  This indicates that the  pages  you

specified can now be used by your task.

1 = Insufficient memory.  When you get this error,

there  were not enough pages free in the system to

accommodate your  request  for  more  memory.   No

additional  pages have been assigned to your task,

even if there were some free memory pages  in  the

system.

(NOTE: Calling the "Available memory" operation to

determine the amount of memory available, followed

by the  "Allocate  memory"  operation  with  fewer

pages than reported to you from "Available memory"

can  still  fail,  since  another  task  may  have

allocated pages in between your two calls.  Do not

rely on being able to call  the  two  routines  in

succession   without   checking   the  error  code

returned from the "Allocate memory" operation.)

7 = Attempt to overwrite shared  page.   You  will

get   this   error   if  any  page  in  the  range

R2..(R2+R1-1) is already allocated  to  your  task

with  a  "shared"  attribute.  No additional pages

have been assigned to your  task  if  you  receive

this  error, even if there were enough free memory

pages in the system to accommodate your request.

8 = Out of table space.   You  will  receive  this

error  if  too  many  tasks  have  large  gaps  of

unassigned  pages  in  their  memory  maps.    The

current  versions  of MDOS allow 480 virtual pages

between all tasks which are  currently  executing.

Note  that  there  are  only 256 possible physical

pages, and that there are only 128 physical  pages

even  if you have the 512k expansion RAM, so tasks

would have to be pretty wasteful (have  more  gaps

than  actual  pages)  in  order  to use up all 480

virtual pages allowed by MDOS.  If  you  get  this

error,  your  program should just give up and tell

the user to try later.

New count           This is the  number  of  pages  which  were  newly

assigned  to  your  task, and is only valid if you

did  not  receive  an  error  from  the  "allocate

memory"  call.   This  number can be less than the

number of pages you requested if some of the pages

in  the  range R2..(R2+R1-1) were already assigned

to your task.

GenREF v1.00                         MEM - 6

Fast count               This is the number of fast pages which were  newly

assigned  to  your  task, and is only valid if you

did  not  receive  an  error  from  the  "allocate

memory" call.  You would use this to check if MDOS

actually assigned any fast pages to your task.

Example 1.1							Filling a hole

R0=1      Opcode

R1=2      Number of pages to get

R2=4      Virtual page number

R3=0      Speed flag

Virtual   Physical  Physical

Page      Page      Page

Before    After

0         >3F       >3F

1         >3E       >3E

2         >3D       >3D

3         (>FF)     (>FF)     hole

4         (>FF)     >33       new page

5         (>FF)     >32       new page

6         (>FF)     (>FF)     hole

7         >3C       >3C

8         >3B       >3B

9         >3A       >3A

10        >39       >39

11        >38       >38

12        >37       >37

13        >36       >36

14        >35       >35

15        >34       >34

Note:                    The pages  (>FF)  represent  holes  in  the  tasks

virtual  memory  map.   The  physical  page >FF is

actually part of the boot rom  on  your  computer,

and cannot be overwritten by your task.

MEM - 7                                  GenREF v1.00

Example 1.2

R0=1      Opcode

R1=1      Number of pages to get

R2=17     Virtual page number

R3=0      Speed flag

Virtual   Physical  Physical

Page      Page      Page

Before    After

0         >3F       >3F

1         >3E       >3E

2         >3D       >3D

3         (>FF)     (>FF)     hole

4         (>FF)     (>FF)     hole

5         (>FF)     (>FF)     hole

6         (>FF)     (>FF)     hole

7         >3C       >3C

8         >3B       >3B

9         >3A       >3A

10        >39       >39

11        >38       >38

12        >37       >37

13        >36       >36

14        >35       >35

15        >34       >34

16        (null)    (>FF)     new hole

17        (null)    >33       new page

GenREF v1.00                         MEM - 8

Notice that this routine only fills holes, it does

not  assign  a new physical page to a virtual page

which is already assigned to your task.

Example 1.3							Overlaying Pages

R0=1      Opcode

R1=5      Number of pages to get

R2=1      Virtual page number

R3=0      Speed flag

Virtual   Physical  Physical

Page      Page      Page

Before    After

0         >3F       >3F

1         >3E       >3E       no change

2         >3D       >3D       no change

3         (>FF)     >33       new page

4         (>FF)     >32       new page

5         (>FF)     >31       new page

6         (>FF)     (>FF)     hole

7         >3C       >3C

8         >3B       >3B

9         >3A       >3A

10        >39       >39

11        >38       >38

12        >37       >37

13        >36       >36

14        >35       >35

15        >34       >34

Notice that even though you  asked  for  5  pages,

only  3  were  actually assigned, since two of the

specified pages had already been assigned.

MEM - 9                             GenREF v1.00

___________________________________________________________________________________

RELEASE MEMORY          ___________________________________________________________________________________

Function            You will use this routine to return unused  memory

to  MDOS, this is useful if your program uses lots

of temporary  data.   This  is  also  one  of  the

functions  used  by  MDOS to free memory when your

task is terminated.  Any page which is released by

your task  which is also currently mapped into one

of  your  task's  seven  memory  windows  will  be

removed  from the memory window used by your task,

and its entry in the mapper will  be  replaced  by

page >FF.

You may not release virtual page zero of your task

using  this  function  (although  page zero may be

accessed by your task, it doesn't really belong to

your task.)

This opcode cannot be used to  free  shared  pages

belonging  to a task.  Shared memory pages must be

freed with opcode #6.

Parameters          R0        = 2 (opcode)

R1        = page count

R2        = starting page

Results             R0        = error code

Parameter description

Page count          This the number of memory pages you wish  to  free

from your program.  It is not necessarily the same

as the number of physical memory pages which  will

actually  be  freed from your task.  Shared pages,

and unallocated pages in  the  range  R2:(R2+R1-1)

will  not  be  released  from your task.  No pages

will be released if this count is zero.

Starting page       This is the  page  number  of  the  first  virtual

memory  page  you  wish to have released from your

task.  This procedure will attempt to release  all

of  your  task's virtual memory pages in the range

R2:(R2+R1-1) into the free page pool.

Error code          0 = No Error.  This indicates that the  non-shared

pages  in  the  range specified have been released

from your task back  to  the  free  pages  in  the

system.

2 = Attempt to free  page  zero.   This  indicates

that  you  tried to free virtual page zero of your

task.  No pages were actually released  from  your

task.

GenREF v1.00                        MEM - 10

8 = Out of table space.  MDOS was unable to free a

page  because  there  weren't enough virtual pages

nodes available to create a new page in  the  free

pool.  When you receive this error, it is possible

that some, but not all, of the pages in the  range

R2:(R2+R1-1)  have  been  moved  to the free pool.

You will receive this error if too many tasks have

large  gaps  of  unassigned  pages in their memory

maps.  The current  versions  of  MDOS  allow  480

virtual   pages   between   all  tasks  which  are

currently executing.  Note that there are only 256

possible  physical  pages, and that there are only

128 physical pages  even  if  you  have  the  512k

expansion  RAM,  so  tasks would have to be pretty

wasteful (have more gaps  than  actual  pages)  in

order  to  use up all 480 virtual pages allowed by

MDOS.  If you get this error, your program  should

just give up and tell the user to try later.

Example 2.1							Making a Hole

R0=2      Opcode

R1=9      Number of pages to release

R2=2      First virtual page to release

Virtual   Physical  Physical

Page      Page      Page

Before    After

0         >3F       >3F

1         >3E       >3E

2         >3D       (>FF)     new hole

3         (>FF)     (>FF)     hole

4         (>FF)     (>FF)     hole

5         (>FF)     (>FF)     hole

6         (>FF)     (>FF)     hole

7         >3C       (>FF)     new hole

8         >3B       (>FF)     new hole

9         >3A       (>FF)     new hole

10        >39       (>FF)     new hole

11        >38       >38

12        >37       >37

13        >36       >36

14        >35       >35

15        >34       >34

Note that only five pages were  actually  released

from your task to MDOS, since some of the pages in

the specified range were already unassigned.

MEM - 11                                    GenREF v1.00

Example 2.2						Making list shorter

R0=2      Opcode

R1=8      Number of pages to release

R2=10     First virtual page to release

Virtual   Physical  Physical

Page      Page      Page

Before    After

0         >3F       >3F

1         >3E       >3E

2         >3D       >3D

3         (>FF)     (>FF)

4         (>FF)     (>FF)

5         (>FF)     (>FF)

6         (>FF)     (>FF)

7         >3C       >3C

8         >3B       >3B

9         >3A       >3A

10        >39       null      freed

11        >38       null      freed

12        >37       null      freed

13        >36       null      freed

14        >35       null      freed

15        >34       null      freed

The list was truncated, since all of the pages  at

the  tail  of  the list were unassigned. Also note

that we really told it to release pages 10 to  18,

but  we  only had pages up to 15 to begin with. No

error is reported  when  you  attempt  to  release

unassigned pages.

GenREF v1.00                           MEM - 12

___________________________________________________________________________________

MAP MEMORY             ___________________________________________________________________________________

Function                 This routine can be used to place a physical  page

into  the  mapper  chip  for the specified virtual

page belonging to your task.  You can think of the

mapper   as  providing  seven  usable  8k  "memory

windows" into your task's  virtual  memory  space.

You  tell  this routine which of the seven windows

to use, and which part of virtual memory  to  look

at.   This  routine  can  not be used to overwrite

page zero of your task.

You should use this routine for mapping memory  if

you   want   your   program  to  be  able  to  use

transparent demand paging in  future  versions  of

MDOS which support page swapping to hard disk.

(NOTE: If you are using window number  seven,  the

one  which  is  at  >E000  in  your direct address

space; the data from offset >1000 to >1140 in  the

page  will  be corrupted by writes to addresses in

the range >F000  to  >F140.   Do  not  use  window

number  seven  unless it is ok for the data in the

specified range to be corrupted.)

On successful return, the specified  virtual  page

belonging  to  your  task has been mapped into the

window you specified and is available for use.

Parameters               R0        = 3 (opcode)

R1        = page number

R2        = window number

Results                  R0        = error code

mapper    = new page

Parameter description

Window number            This parameter, in  the  range  1:7,  is used to

tell  MDOS  which  of  the seven 8k byte windows

in the processor's 16-bit address space to use for

the  specified virtual page belonging to your task.

Processor address = Window_number * >2000

Page number              This is the virtual page number within  your  task

that  you  wish  to have mapped into the specified

window in the processor's  16-bit  address  space.

Virtual  memory  in  the  address  range  (page  *

>2000):(page * >2000 +>1fff) will be accessible to

your  program  with the 16-bit addresses (window *

>2000):(window * >2000 + >1fff).

MEM - 13                                  GenREF v1.00

Error code          0 = No Error.  This indicates that  the  specified

virtual  memory  page of your task has been mapped

into the specified memory window.

2 = Header page mapping violation.  You  attempted

to  map  a virtual page into window zero, which is

reserved  for  your   task's   header   by   MDOS.

Alternatively,  you  attempted to map virtual page

zero, your task's header, into some  other  memory

window.   Your  task's  memory  map  has  not been

changed if you get this error.

3 = Unassigned virtual  page,  or  Invalid  memory

window.   The virtual page you specified has never

been allocated by your task, and contains no valid

data  (it  is  a  "hole"  in  your address space.)

Alternatively,  you  specified  a  window   number

larger than seven.  Your task's memory map has not

be changed if you get this error.

Mapper              In MDOS mode, the mapper  is  8  bytes  long,  and

located at >F110 in the processor's 16-bit address

space.   Each  byte  in  the  mapper  contains   a

physical memory page number, in the range >00:>FF.

Assignments are as follows:

Mapper register = >F110 + Window_number

Note that each mapper register  corresponds  to  a

specific   8k  block  in  the  processor's  16-bit

address space.  After successful completion of the

"Map   Memory"   function,   the  mapper  register

corresponding to the  window  you  specified  will

contain  the  physical  page number of the virtual

page you specified.

Symbolically:

mapper[window number]  =  task_pages[virtual  page

number]

GenREF v1.00                           MEM - 14

___________________________________________________________________________________

GET MEMORY LIST               ___________________________________________________________________________________

Function                 This operation returns an array of  physical  page

numbers  (each  is  one byte) corresponding to the

virtual pages belonging to  your  task.   You  are

allowed  to  specify the address of the first byte

in the array, and the maximum number  of  elements

in the array.

This array of physical page numbers is  useful  if

you need to speed up memory paging in your task by

use of your own  paging  code.   If  your  program

performs  its  own  paging,  you will need to call

this  opcode  after  every  call   to   a   memory

management  function  which  adds or removes pages

from your virtual address space. (Opcodes 1,2,6,7)

In  future  versions   of   MDOS   which   support

transparent  demand  paging,  this  operation will

also "lock" all of your task's virtual pages  into

RAM,   making  them  ineligible  for  paging.   To

"unlock" your virtual pages, another  opcode  will

be  provided  for  your use. (Normally, only pages

which are currently mapped into one of  the  seven

processor  windows for your task would be "locked"

into RAM.)

On successful return, the array will  contain  the

physical  page  numbers  for  the virtual pages in

your task.

Parameters               R0        = 4 (opcode)

R1        = array start

R2        = array size

Results                  R0        = error code

R1        = array used

Parameter description

Array start              You specify the address of the first byte in  your

array  using  this  parameter.   The physical page

number of your task's header  page,  virtual  page

number  zero,  is  placed in the first byte of the

array.  This is a 16-bit processor address  for  a

location  which  is  currently  mapped  into  your

task's memory windows.

Array size               This  is  the  maximum  number  of  physical  page

numbers  which  can be returned to your task.  The

indexes of the array elements can range from  zero

to (array size)-1.

MEM - 15                                  GenREF v1.00

Error code       0 = No  Error.   This  indicates  that  the  array

contains all of the physical page associations for

the virtual pages in your  task.   The  number  of

actual  array  elements  used can be less than the

maximum size you specified for the array.

8 = Array not large enough.  Your  array  was  not

large  enough  to  hold  all  of the physical page

numbers in use by your task.  When  you  get  this

error,  the  contents of the array are valid up to

the maximum element which you allowed.

Array used       This indicates the number of valid pages  returned

in  the  array.  When you perform your own paging,

you should make sure that you never index into the

array  after  the last valid page (You will end up

mapping a page which doesn't belong to your task.)

Sample Code

Assuming that you've already called  this  opcode,

the  follow  code fragment will map in a data item

pointed to by a 32 bit address.

assume: r1,r2 = 32 bit pointer, @paglst are bytes from opcode #4

movb r2,r1         ok, since only low 5 bits of r1 are used

andi r1,>e01f      keep 8 bits, zap the others

src  r1,13         rotate to make an index into the page list

andi r2,>1fff      mask off the high three bits, they're now

*                   in R1 ...

*

movb @paglst(r1),@mapper+4      put it at >8000

movb @paglst+1(r1),@mapper+5    put next page at >a000

*

* it is not necessary to place two pages into the mapper if you

* know for certain that the record accessed by the pointer does

* not cross page boundaries, the above code is just a method of

* playing it safe

*

mov  @>8000+field_offset(r2),r3

*

* this of course assumes that there is some record addressed by

* the initial pointer, and that the record contains fields of some

* data structure.  Fields are easy to set up with a DORG statement

* for each record type in use by an application

*

GenREF v1.00                          MEM - 16

___________________________________________________________________________________

DECLARE SHARED MEMORY               ___________________________________________________________________________________

Function                 This routine is used to declare a range  of  pages

currently  belonging  to  your  task  as  "shared"

memory pages, which means that they can be used by

other  tasks.   An  example  of  two tasks sharing

memory would be an editor which  "shared"  all  of

its  text  buffer  with  an assembler, so that the

assembler could assemble from RAM rather than from

disk.

Each group of pages declared as shared has a type,

which  you assign.  When another application wants

to share those memory pages  with  your  task,  it

will  ask  MDOS  to  use  a certain type of shared

pages.  An editor buffer could be declared as  one

specific type, while object code would be declared

as a separate type, so that programs would not use

the wrong sort of data as input (You wouldn't want

a Fortran compiler to use a binary program as  its

input!)

It is recommended that "types" be assigned by  the

distributor   of   MDOS,   so   that  incompatible

applications do not try to use the same "type"  if

you  decide to use a "type" please correspond with

the  distributor  of  MDOS  to   coordinate   your

development efforts with others.

A "shared" type may only  be  declared  once,  and

always  resides  in a group of consecutive virtual

pages.  If all applications using a "shared" group

of  pages  release  those pages, the "type" may be

redeclared.  (MDOS keeps a count of the number  of

applications  using  a  shared  group,  and if the

count ever becomes zero, the type is made free for

re-use)

Note: It is not possible to declare page 0  to  be

part of a shared group.  Page 0 is always private,

since it contains the information which MDOS  uses

to distinguish between tasks.

Parameters               R0        = 5 (opcode)

R1        = page count

R2        = starting page

R3        = shared type

Results

R0        = error code

MEM - 17   	                           GenREF v1.00

Parameter Description

Page count          This is the number of  consecutive  virtual  pages

belonging to your task which will be declared as a

shared page group for use by other tasks.

Starting page       This is the virtual page number within  your  task

of  the  first virtual page which will become part

of the shared page  group.   Pages  in  the  range

(start_page):(start_page+page_count-1) will belong

to the group.

Shared type         This must be in the range >01:>FE, and should be a

code  unique  to  the format of data which you are

sharing with other tasks.  It is recommended  that

you  use  a common set of source code routines for

data access for all of your tasks  which  use  the

data.

Error code          0 = No Error.  This indicates that  virtual  pages

you  specified  can  now  be shared by other tasks

running under MDOS.

3 = Bad page.  At least one of the  pages  in  the

range  (start_page):(start_page+page_count-1)  has

never been allocated by  your  task.   The  shared

group has not been defined if you get this error.

5  =  Invalid  type  code.   Your  "shared   type"

parameter  was  not  in the range >01:>FE, or your

"shared type" code has already  been  declared  by

another  task.   The shared group does not contain

the pages you specified if you get this error.

7 = Invalid page declaration.  At least one of the

pages in the range (start_page) : (start_page +

page_count-1)is unallocated,  already  declared as

shared, or is virtual page number zero. The shared

group has not been defined if you get this error.

8 = Out of table space.  MDOS was unable to create

the  shared  type  because  weren't enough virtual

pages nodes available  to  create  a  shared  page

group descriptor list. You will receive this error

if too many tasks have large  gaps  of  unassigned

pages  in their memory maps.  The current versions

of MDOS allow 480 virtual pages between all  tasks

which  are  currently  executing.  Note that there

are only 256 possible  physical  pages,  and  that

there are only 128 physical pages even if you have

the 512k expansion RAM, so tasks would have to  be

pretty wasteful (have more gaps than actual pages)

in order to use up all 480 virtual  pages  allowed

by  MDOS.   If  you  get  this error, your program

should just give up  and  tell  the  user  to  try

later.

GenREF v1.00                           MEM - 18

___________________________________________________________________________________

RELEASE SHARED MEMORY          ___________________________________________________________________________________

Function            This operation removes all shared memory pages  of

the specified type from your task's virtual memory

list.  If your task was the only  task  using  the

shared   page   group,   the   group  will  become

undefined, and must be redeclared before use.  Any

page which is released by your task  which is also

currently mapped into one  of  your  task's  seven

memory  windows  will  be  removed from the memory

window used by your task, and  its  entry  in  the

mapper will be replaced by page >FF.

Parameters          R0        = 6 (opcode)

R1        = shared type

Results             R0        = error code

Parameter description

Shared type         This is a shared group type number, in  the  range

>01:>FE,  and must have been previously defined by

another task.

Error code          0 = No Error.  This  indicates  that  all  of  the

pages  from  the  shared  group you specified have

been released from your task.

6 = Invalid type.  The type you specified was  not

in  the range >01:>FE, or hasn't yet been declared

by another task.

8 = Out of table space.  MDOS was unable to free a

page  because  there  weren't enough virtual pages

nodes available to create a new page in  the  free

pool.  When you receive this error, it is possible

that some, but not all, of the pages belonging  to

the shared group have been moved to the free pool.

If you get this error, your  program  should  just

give up and tell the user to try later.

GenREF v1.00                           MEM - 19

___________________________________________________________________________________

USE SHARED MEMORY               ___________________________________________________________________________________

Function                 This operation will include  the  pages  from  the

shared  type  specified  in  your  task's  list of

virtual pages.   The  shared  type  must  be  been

previously  declared  by  another  task.  When you

call  this  function,  all  pages  in  the   range

(start_page):(start_page+shared_size-1)  must  not

be allocated by  your  task,  since  you  are  not

permitted  to overlay shared and existing pages in

your virtual page list.

Parameters

R0        = 7 (opcode)

R1        = shared type

R2        = start page

Results                  R0        = error code

Parameter Description

Shared type              This is a type code for a shared page group  which

must  have been declared by another task.  If your

task has enough contiguous available virtual pages

beginning  with  the start page you specified, all

of the pages from the shared page  group  will  be

mapped in at the specified virtual page address.

Start page               This is the virtual page number within  your  task

of  the  first  virtual page which will be used by

the  shared  page  group.   After   calling   this

operation,  you must explicitly map in the virtual

pages which have just been  assigned,  since  they

will  not be automatically placed into your task's

mapper registers.

Error code               0 = No Error.  The shared page group of  the  type

you   requested   was   already  defined  and  was

successfully mapped into your task's virtual  page

list.

2 = Attempt to overlay page zero.   You  specified

virtual page zero as the start page for the shared

memory group.  No pages from the memory group have

been allocated to your task if you get this error.

6 = Invalid shared type.  The type  you  specified

was  not  in the range >01:>FE or has not yet been

defined for use by another task.   No  pages  from

the  memory group have been allocated to your task

if you get this error.

MEM - 20                                GenREF v1.00

7 = Attempt to overlay shared and private  memory.

Your  task  did  not  have  enough contiguous free

virtual  pages  starting  with  the  virtual  page

specified  to  map  in  the  pages from the shared

group.  No pages from the memory group  have  been

allocated to your task if you get this error.

8 = Out of table space.   There  were  not  enough

free nodes available to extend your task's virtual

page list.  MDOS is out of table space.   At  this

point,  your task should give up and tell the user

to try later.  No pages from the memory group have

been allocated to your task if you get this error.

___________________________________________________________________________________

GET SIZE OF SHARED GROUP          ___________________________________________________________________________________

Function            This operation reports the number of  pages  which

belong  to a shared page group.  It should be used

by your task before you have the shared page group

assigned  into your virtual page list, so that you

know in advance if your  task  has  enough  unused

contiguous pages to overlay the shared page group.

Parameters          R0        = 8 (opcode)

R1        = shared type

Results             R0        = error code

R1        = shared size

Parameter description

Shared type         This is a type code for a shared page group  which

must  have  been  declared  by  another  task.  On

successful return, the number  of  pages  in  this

page group is returned to your task.

Error code          0 = No Error.  The shared page group of  the  type

you requested was already defined and its size was

returned to you.

6 = Invalid shared type.  The type  you  specified

was  not  in the range >01:>FE or has not yet been

defined for use by another task.

Shared size         On successful return, this will contain the  size,

in 8k pages, of the specified shared page group.

GenREF v1.00                          MEM - 21

___________________________________________________________________________________

FREE TASK               ___________________________________________________________________________________

Function                 This routine can not be  directly  used  by  tasks

under  MDOS,  it  is  reserved  for  use by system

library functions.

This is used to free all memory pages, except  the

task's  header,  from  the  task's list of virtual

pages.  If the task is using a shared page  group,

its  reference  to  the group will be removed, and

the group itself will be removed if this  was  the

last task using the shared page group.

Parameters               R0        = 9 (opcode)

R1        = first node

Results                  R0        = error code

Parameter description

First node               This is the MEMLST pointer from the task's header.

Error code               0 = No Error.  The pages  belonging  to  the  task

were freed.

>FFFF = Invalid opcode.   You  attempted  to  call

this from a user task.

___________________________________________________________________________________

GET MEMORY PAGE               ___________________________________________________________________________________

Function                 This routine can not be  directly  used  by  tasks

under  MDOS,  it  is  reserved  for  use by system

library functions.

It is  used  to  get  a  single  memory  page,  by

specific   physical   page  number,  or  by  speed

priority.

Parameters               R0        = 10 (opcode)

R1        = physical page

R2        = speed flag

Results                  R0        = error code

R1        = node pointer

MEM - 22                               GenREF v1.00

Parameter description

Physical page       If this is in the range >00:>FF, MDOS will  return

a pointer to the memory node for the page, only if

the page is currently unassigned.

If this is larger than >FF,  MDOS  will  return  a

pointer  to the memory node for the the first free

page  in  the  system  with  the  specified  speed

attribute.

Speed flag          This parameter is used only if the  physical  page

number  specified  is larger than >FF.  If this is

zero, MDOS will allocate  the  first  memory  page

available  in the free list.  If this is non-zero,

MDOS will attempt to allocate the first zero  wait

state  page available from the free list, if there

are no zero wait state pages available, MDOS  will

allocate the first free page it finds.

Error code          0 = No Error.  The page was reserved as specified,

it  is  not  assigned  to  any task, and it is not

available for use.

1 = Page not available.  The  specified  page  was

not free, or there are no free pages in the entire

system.

>FFFF = Invalid opcode.   You  attempted  to  call

this from a user task.

Node pointer        This is pointer to a 4 byte memory node inside  of

the memory library's address space.

___________________________________________________________________________________

FREE MEMORY PAGE          ___________________________________________________________________________________

Function            This routine can not be  directly  used  by  tasks

under  MDOS,  it  is  reserved  for  use by system

library functions.

Adds the specified physical page to  the  list  of

pages available for use by user tasks.

Parameters          R0        = 11 (opcode)

R1        = page number

Results             R0        = error code

Parameter description

Page number         This a simply a physical page number to be freed.

GenREF v1.00                        MEM - 23

Error code               0 = No Error.  The page was reserved as specified,

it  is  not  assigned  to  any task, and it is not

available for use.

8 = Out of table space.  MDOS was unable to create

a  free  page because there weren't enough virtual

pages nodes available to create a new page in  the

free pool.

>FFFF = Invalid opcode.   You  attempted  to  call

this from a user task.

___________________________________________________________________________________

FREE MEMORY NODE

___________________________________________________________________________________

Function                 This routine can not be  directly  used  by  tasks

under  MDOS,  it  is  reserved  for  use by system

library functions.

This operation will add the specified 4-byte  node

to the memory nodes available for use by the other

memory management library functions.

Paramaters               R0        = 12 (opcode)

R1        = node address

Results                  R0        = error code

Parameter description

Node address             This is the address of the 4-byte node within  the

memory management library's address space.

Error code               0 = No Error.  The node was added to the free node

list.

>FFFF = Invalid opcode.   You  attempted  to  call

this from a user task.

___________________________________________________________________________________

LINK MEMORY NODE               ___________________________________________________________________________________

Function                 This routine can not be  directly  used  by  tasks

under  MDOS,  it  is  reserved  for  use by system

library functions.

This is used to link memory  nodes  together.   It

can  be  used  to  link  page  nodes onto a task's

virtual memory list, to link  page  nodes  to  the

system  free page list, and to link nodes into the

free node list.

MEM - 24                          GenREF v1.00

Parameters          R0        = 13 (opcode)

R1        = new node

R2        = old node

Results             R0        = error code

Parameter description

New node            The node to be inserted into a list after the  old

node.

Old node            The node, in a node list, after which the new node

is to be inserted.

Error code          0 = No Error.  The nodes were linked together.

>FFFF = Invalid opcode.   You  attempted  to  call

this from a user task.

___________________________________________________________________________________

GET MEMORY LIST (system)          ___________________________________________________________________________________

Function            This routine can not be  directly  used  by  tasks

under  MDOS,  it  is  reserved  for  use by system

library functions.

This routine will return  a  task's  virtual  page

list  to location >1F00 in system page zero.  This

is used primary by the DSR routines to locate data

pointed to by a task's PAB buffer address.

Parameters          R0        = 14 (opcode)

Results             R0        = -1 (error code)

R0        = page count (no error)

Parameter description

Error code          >FFFF = Invalid opcode.   You  attempted  to  call

this from a user task.

Page count          This is the number of valid pages in the page list

at  >1F00 in system page zero.  This count is also

returned at >1FFE.