*  LGEN/S

*  ~~~~~~

*  XB LOAD PROGRAM GENERATOR

*  A FUNNELWEB FARM UTILITY

*  -------------------------

*

*  INSTRUCTIONS

*  ~~~~~~~~~~~~

*

*  Bring up Replace String (RS)

*

*  Enter /XXXX/yyyy/ where yyyy

*   is the 4 digit hex address

*   for the start of the assembly

*   language area

*

*  SaveFile to disk under working

*   file name LG/S and assemble

*   with R option only as file LG

*

*  Clear out top of hi-mem above

*   DEBUG (optional) and QUIT

*   directly (not by .Q)

*

*  NEW out XB

*

*  RUN the following program

*

*   100 CALL INIT :: CALL LOAD

*   ("LG"):: CALL LINK("START")

*

*  SAVE to disk

*

*  OLD to check SIZE and fill

*   with machine code routine

*

*  Do NOT RESequence directly

*

*  Always acknowledge origin

*   of this program.

IDT  'LOADGEN'

LNTAB  EQU  >XXXX->56

DEF  START

A      DATA >0064        Line #100

DATA LNTAB+5      Pointer to line

BYTE >51          Length byte for XB line

BYTE >83          XB ! token

TEXT 'FUNNELWEB FARM TEMPLATE'

TEXT 'AVAILABLE ADDRESSES ABOVE XB'

TEXT '~~~~  >XXXX  TO  >FFD7  ~~~~'

BYTE 0

B      EQU  $

EVEN

DATA >0,>0,>0,>0  Make end obvious

C      EQU  $

START  EQU  $

LI   R0,LNTAB     Set XB's pointers

MOV  R0,@>8330    Start of line # table

MOV  R0,@>832E    Start of line # table

AI   R0,3

MOV  R0,@>8332    End of line # table

LI   R0,A         Write to place

LI   R1,LNTAB

MV1    MOVB *R0+,*R1+

CI   R0,C

JL   MV1

MOVB @B,@>837C    Clear GPL status

RT                Return to XB

END