*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*          Low level power up routine for SCSI DSR          *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

*-----------------------------------------------------------

* NAME : PWRUP

*

* DESC : INITIALISES SCSI CARD AND RESETS SCSI DEVICES

*

* PARM : R12 = CRU BASE OF SCSI CARD

*

* RET  : NOTHING

*

* DEST : R0,R1,R2

PWRUP  MOV  R11,R2            SAVE RETURN ADDRESS

LI   R0,>000D          INITIAL CRU SETTINGS

LDCR R0,0              INITIALISE SCSI CARD CRU

MOVB R0,@WMR2          NO SCSI INTERRUPTS

MOVB R0,@WSER          NO RESELECTION

MOVB @RRPI,R1          CLEAR ANY PENDING INTERRUPTS

LI   R1,ICRRST*256     PREPARE TO RESET SCSI BUS

MOVB R1,@WICR          ASSERT RST*

BL   @RSTDEL           DELAY FOR RESET

MOVB R0,@WICR          DEASSERT RST*

PWRUP1 TB   IRQ               WAIT FOR RST* INTERRUPT

JNE  PWRUP1            NO INTERRUPT YET

MOVB @RRPI,R1          CLEAR INT LATCHES

* INITIALISE THE SECTOR SIZE TABLE

LI   R0,SECLTB         POINTER TO SECTOR SIZE TABLE

LI   R1,8              8 ENTRIES TO CLEAR

PWRUP2 CLR  *R0+              CLEAR ENTRY

DEC  R1                CLEARED ALL 8 YET ?

JNE  PWRUP2            NO

B    *R2               RETURN