*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*   Delay routines that may need to be tweaked for Geneve   *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

*-----------------------------------------------------------

* NAME : RSTDEL,ARBDEL,BSDEL,BCDEL

*

* DESC : VARIOUS DELAYS REQUIRED BY THE SCSI SPECIFICATION

*

* PARM : R12 = CRU BASE OF SCSI CARD

*

* RET  : R0 = ZERO ALWAYS

*

* DEST : R0

*

* NOTE : THESE DELAYS HAVE BEEN KEPT SEPARATE FROM THE REST

*        OF THE CODE TO ALLOW FOR VARIATIONS IN CPU SPEED.

*        FOR EXAMPLE, THE COUNTS WILL BE GREATER FOR GENEVE.

RSTDEL LI   R0,RSTCNT         RESET ASSERTION DELAY

JMP  DELAY

ARBDEL LI   R0,ARBCNT         ARBITRATION DELAY

JMP  DELAY

BSDEL  LI   R0,BSCNT          BUS SETTLE DELAY

JMP  DELAY

BCDEL  LI   R0,BCCNT          BUS CLEAR DELAY

JMP  DELAY

SELDEL LI   R0,BSCNT+BCCNT    BUS SETTLE + BUS CLEAR DELAY

DELAY  DEC  R0                DELAY LOOP

JNE  DELAY

B    *R11              RETURN