*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*              Low level SCSI code for NCR5380              *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

*-----------------------------------------------------------

* NAME : ARBIT

*

* DESC : ARBITRATE FOR THE SCSI BUS

*

* PARM : NONE

*

* RET  : R0 = ZERO ALWAYS

*

* DEST : R0,R1,R2,R3

*

* NOTE : DOES NOT RETURN UNTIL ARBITRATION SUCCESSFUL

ARBIT  MOV  R11,R3            SAVE RETURN ADDRESS

ARBIT0 CLR  R0                HANDY ZERO

MOVB @IDMASK,R1        CONTROLLER ID MASK

MOVB R1,@WODR          CONTROLLER ID MASK -> BUS

MOVB R0,@WMR2          CLEAR ARB

LI   R2,MR2ARB*256     PREPARE TO ARBITRATE

MOVB R2,@WMR2          INITIATE ARBITRATION

ARBIT1 MOVB @RICR,R2          READ ICR

ANDI R2,ICRAIP*256     AIP SET ( BUS FREE ) ?

JEQ  ARBIT1            NO, BUS NOT FREE YET

BL   @ARBDEL           YES, 2.2us ARBITRATION DELAY

MOVB @RICR,R2          READ ICR

ANDI R2,ICRLA*256      LA SET ( LOST ARBITRATION ) ?

JNE  ARBIT0            YES, ARBITRATE AGAIN

MOVB @RCSD,R2          NO, READ SCSI BUS DATA

XOR  R1,R2             CLEAR CONTROLLER ID BIT

CB   R2,R1             ANY HIGHER ID'S ?

JH   ARBIT0            YES, ARBITRATE AGAIN

B    *R3               NO, WE WON ARBITRATION

*------------------------------------------------------------

* NAME : SELECT

*

* DESC : SELECTS A SCSI TARGET UNIT

*

* PARM : R0  = SCSI TARGET ID TO SELECT

*

* RET  : R0 = ZERO IF UNIT SELECTED

*           = NON-ZERO ERROR CODE IF PROBLEM

*

* DEST : R0,R1,R2,R3,R4,R5

ICRBSD EQU  ICRBSY+ICRSEL+ICRBUS

ICRSD  EQU  ICRSEL+ICRBUS

SELECT MOV  R11,R5            SAVE RETURN ADDRESS

CI   R0,7              TARGET ID > 7 ?

JH   SEL3              YES, ERROR

MOVB @IDNUM,R1         NO, FETCH CONTROLLER ID

SRL  R1,8              TO LSB

C    R0,R1             TARGET ID == CONTROLLER ID ?

JEQ  SEL3              YES, ERROR

LI   R4,>0100          NO, DEFAULT ID 0 MASK

MOV  R0,R0             TARGET ID = 0 ?

JEQ  SEL1              YES, USE DEFAULT MASK

SLA  R4,0              NO, CREATE MASK FOR ID

SEL1   SOCB @IDMASK,R4        OR WITH CONTROLLER ID MASK

BL   @ARBIT            ARBITRATE FOR THE BUS

LI   R2,ICRSEL*256     PREPARE TO ASSERT SEL*

MOVB R2,@WICR          ASSERT SEL*

MOVB R0,@WTCR          CLEAR TCR ( NO RESELECTION )

MOVB R4,@WODR          LOAD ID'S FOR ASSERTION ONTO BUS

LI   R1,ICRBSD*256     PREPARE TO ASSEY BSY*, SEL* AND DBUS

MOVB R1,@WICR          ASSERT BSY*, SEL* AND DBUS

MOVB R0,@WMR2          CLEAR ARB

LI   R1,ICRSD*256      PREPARE TO DEASSERT BSY*

MOVB R1,@WICR          DEASSERT BSY*

BL   @SELDEL           WAIT A SELECT DELAY

LI   R1,BSYCNT         WAIT COUNT FOR BSY* DETECTION

SEL2   MOVB @RCSB,R2          READ CSB

ANDI R2,CSBBSY*256     IS BSY* ASSERTED ?

JNE  SEL4              YES, TARGET HAS BEEN SELECTED

DEC  R1                TIMEOUT ?

JNE  SEL2              NO, KEEP WAITING FOR BSY*

SEL3   SETO R0                SET NON-ZERO RETURN CODE FOR ERROR

SEL4   CLR  R1                SAVE MEMORY ACCESSES

MOVB R1,@WICR          DEASSERT SEL* AND DBUS*

B    *R5               RETURN

*------------------------------------------------------------

* NAME : NWRITE

*

* DESC : NON-DMA WRITE DATA TO SELECTED UNIT

*

* PARM : R0  = POINTER TO DATA TO WRITE

*        R1  = LENGTH OF DATA IN BYTES

*        R2  = SCSI PHASE OF TRANSFER

*

* RET  : R0 = NUMBER OF BYTES ACTUALLY TRANSFERRED

*

* DEST : R0,R1,R2,R3,R4,R5,R6,R7

ICRBA  EQU  ICRBUS+ICRACK

NWVINS EQU  >D592             MOVB *R2,*R6

NWCINS EQU  >D5B2             MOVB *R2+,*R6

* ENTRY POINT FOR WRITE FROM VDP RAM

NWRITV MOV  R11,R7            SAVE RETURN ADDRESS

BL   @SETRDA           SET VDP READ ADDRESS

LI   R0,VDPRD          VDP READ ADDRESS

LI   R5,NWVINS         INSTRUCTION TO READ FROM VDP RAM

JMP  NWRIT0            ENTER WRITE LOOP

* ENTRY POINT FOR WRITE FROM CPU RAM

NWRITC MOV  R11,R7            SAVE RETURN ADDRESS

LI   R5,NWCINS         INSTRUCTION TO READ FROM CPU RAM

NWRIT0 LI   R6,WODR           SECOND HALF OF INSTRUCTION

SWPB R2                EXPECTED PHASE TO MSB

MOVB R2,@WTCR          LOAD PHASE INTO TCR

LI   R4,ICRBUS*256     PREPARE TO ASSERT DATA BUS

MOVB R4,@WICR          ASSERT DATA BUS

MOV  R0,R2             COPY DATA POINTER

CLR  R0                CLEAR BYTE TRANSFER COUNTER

MOV  R1,R1             ZERO LENGTH SPECIFIED ?

JEQ  NWRIT3            YES, SAFETY NET EXIT

NWRIT1 MOVB @RCSB,R3          READ CSB

ANDI R3,CSBREQ*256     REQ* ACTIVE ?

JEQ  NWRIT1            NO, WAIT FOR REQ* ACTIVE

MOVB @RBSR,R3          YES, READ BSR

ANDI R3,BSRPHM*256     PHASE MATCH ?

JEQ  NWRIT3            NO, PHASE ERROR

X    R5                WRITE BYTE FROM VDP OR CPU RAM

LI   R3,ICRBA*256      PREPARE TO ASSERT ACK* AND DBUS

MOVB R3,@WICR          ASSERT ACK* AND DATA ONTO BUS

NWRIT2 MOVB @RCSB,R3          READ CSB

ANDI R3,CSBREQ*256     REQ* INACTIVE ?

JNE  NWRIT2            NO, WAIT FOR REQ* INACTIVE

MOVB R4,@WICR          DEASSERT ACK* AND DBUS

INC  R0                INCREMENT TRANSFER COUNTER

DEC  R1                WRITTEN ALL BYTES YET ?

JNE  NWRIT1            NO, MORE LEFT TO WRITE

NWRIT3 B    *R7               YES, RETURN TRANSFER COUNT

*------------------------------------------------------------

* NAME : NREAD

*

* DESC : NON-DMA READ DATA FROM SELECTED UNIT

*

* PARM : R0  = POINTER TO DATA BUFFER

*        R1  = LENGTH OF DATA IN BYTES

*        R2  = SCSI PHASE OF TRANSFER

*

* RET  : R0 = NUMBER OF BYTES ACTUALLY TRANSFERRED

*

* DEST : R0,R1,R2,R3,R5,R6,R7

NRVINS EQU  >D496             MOVB *R6,*R2

NRCINS EQU  >DC96             MOVB *R6,*R2+

* ENTRY POINT FOR READ INTO VDP RAM

NREADV MOV  R11,R7            SAVE RETURN ADDRESS

BL   @SETWDA           SET VDP WRITE ADDRESS

LI   R0,VDPWD          VDP WRITE ADDRESS

LI   R5,NRVINS         INSTRUCTION TO READ FROM VDP RAM

JMP  NREAD0            ENTER READ LOOP

* ENTRY POINT FOR WRITE FROM CPU RAM

NREADC MOV  R11,R7            SAVE RETURN ADDRESS

LI   R5,NRCINS         INSTRUCTION TO READ FROM CPU RAM

NREAD0 LI   R6,RCSD           REST OF READ INSTRUCTION

SWPB R2                EXPECTED PHASE TO MSB

MOVB R2,@WTCR          LOAD PHASE INTO TCR

MOV  R0,R2             COPY DATA POINTER

CLR  R0                CLEAR BYTE TRANSFER COUNTER

MOV  R1,R1             ZERO LENGTH SPECIFIED ?

JEQ  NREAD3            YES, SAFETY NET EXIT

NREAD1 MOVB @RCSB,R3          READ CSB

ANDI R3,CSBREQ*256     REQ* ACTIVE ?

JEQ  NREAD1            NO, WAIT FOR REQ* ACTIVE

MOVB @RBSR,R3          YES, READ BSR

ANDI R3,BSRPHM*256     PHASE MATCH ?

JEQ  NREAD3            NO, PHASE ERROR

X    R5                READ BYTE INTO VDP OR CPU RAM

LI   R3,ICRACK*256     PREPARE TO ASSERT ACK*

MOVB R3,@WICR          ASSERT ACK*

NREAD2 MOVB @RCSB,R3          READ CSB

ANDI R3,CSBREQ*256     REQ* INACTIVE ?

JNE  NREAD2            NO, WAIT FOR REQ* INACTIVE

CLR  R3                PREPARE TO DEASSERT ACK*

MOVB R3,@WICR          DEASSERT ACK*

INC  R0                INCREMENT TRANSFER COUNTER

DEC  R1                READ ALL REQUESTED BYTES YET ?

JNE  NREAD1            NO, MORE LEFT TO WRITE

NREAD3 B    *R7               YES, RETURN TRANSFER COUNT