*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*             SCSI device interrogation routine             *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

* RDYCNT IS THE TIMEOUT USED TO WAIT FOR DRIVE READY

RDYCNT EQU  100

*------------------------------------------------------------

* NAME : INTERG

*

* DESC : WAIT FOR UNIT READY AND FIND SECTOR SIZE

*

* PARM : R0 = TARGET UNIT

*

* RET  : R0 = ZERO IF SUCCESSFUL

*           = NON-ZERO IF FAILURE

*        R1 = SECTOR SIZE IF SUCCESSFUL

*

* DEST : R0,R1,R2,R3,R4,R5,R6,R7,R8,R10

INTERG DATA LLWS1,INTRG0      BLWP VECTOR FOR ROUTINE

* ISSUE TEST READY COMMANDS UNTIL READY OR TIMEOUT

INTRG0 LI   R8,RDYCNT         TIMEOUT COUNTER

INTRG1 MOV  *R13,R0           FETCH UNIT NUMBER FROM OLD R0

BL   @SELECT           SELECT TARGET UNIT

MOV  R0,R0             SELECT OK ?

JNE  INTRG4            NO, SELECTION FAILED

LI   R0,RSECMD         YES, POINTER TO REQUEST SENSE CMD

LI   R1,6              6 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           SEND COMMAND TO TARGET UNIT

CI   R0,6              ALL BYTES TRANSFERRED ?

JNE  INTRG1            NO, TRY AGAIN

LI   R0,CDBBUF         BUFFER POINTER FOR SENSE DATA

LI   R1,4              6 BYTES OF SENSE DATA

LI   R2,PHDATI         DATA IN PHASE

BL   @NREADC           READ SENSE DATA FROM TARGET

LI   R0,CDBBUF+6       BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,CDBBUF+6       BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

MOVB @CDBBUF+2,R0      FETCH BYTE WITH SENSE KEY

ANDI R0,>0F00          MASK OUT ALL BUT SENSE KEY

JEQ  INTRG2            ALL OK IF SENSE KEY IF ZERO

DEC  R8                NOT READY, TIMEOUT ?

JNE  INTRG1            NO

JMP  ITGRDE            YES, DRIVE NOT READY

* ISSUE A SCSI INQUIRY COMMAND TO FIND DEVICE TYPE

INTRG2 MOV  *R13,R0           FETCH UNIT NUMBER FROM OLD R0

BL   @SELECT           SELECT TARGET UNIT

MOV  R0,R0             SELECT SUCCESSFUL ?

INTRG4 JNE  ITGSLE            NO, SELECTION FAILED

LI   R0,INQCMD         POINTER TO INQUIRY COMMAND

LI   R1,6              6 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           SEND COMMAND TO TARGET

CI   R0,6              ALL CDB BYTES SENT ?

JNE  INTRG2            NO, TRY AGAIN

LI   R0,CDBBUF         BUFFER FOR INQUIRY DATA

LI   R1,2              2 BYTES OF INQUIRY DATA TO READ

LI   R2,PHDATI         DATA IN PHASE

BL   @NREADC           READ INQUIRY DATA

LI   R0,CDBBUF+2       BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,CDBBUF+3       BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

MOVB @CDBBUF+2,R0      BAD STATUS ?

JNE  ITGRCE            YES, RETURN ERROR

* CHECK THE DEVICE TYPE AND ONLY ALLOW DIRECT ACCESS + CDROMS

MOVB @CDBBUF,R0        FETCH DEVICE TYPE BYTE

SLA  R0,3              KILL 3 MS BITS

SRL  R0,11             DEVICE TYPE TO LSB

JEQ  INTRG3            IF ZERO WE HAVE DIRECT ACCESS

CI   R0,5              IS DEVICE A CDROM ?

JNE  ITGTPE            NO, RETURN ERROR

* ISSUE A READ CAPACITY COMMAND TO DETERMINE BLOCK SIZE

INTRG3 MOV  *R13,R0           FETCH UNIT NUMBER FROM OLD R0

BL   @SELECT           SELECT TARGET UNIT

MOV  R0,R0             SELECT SUCCESSFUL ?

JNE  ITGSLE            NO, SELECTION FAILED

LI   R0,RCPCMD         POINTER TO READ CAPACITY COMMAND

LI   R1,10             10 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           SEND COMMAND TO TARGET

CI   R0,10             ALL CDB BYTES SENT ?

JNE  INTRG3            NO, TRY AGAIN

LI   R0,CDBBUF         BUFFER FOR CAPACITY DATA

LI   R1,8              8 BYTES OF CAPACITY DATA TO READ

LI   R2,PHDATI         DATA IN PHASE

BL   @NREADC           READ CAPACITY DATA

LI   R0,CDBBUF+8       BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,CDBBUF+9       BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

MOVB @CDBBUF+8,R0      BAD STATUS ?

JNE  ITGRCE            YES, RETURN ERROR

* EXTRACT THE BLOCK SIZE FROM RETURNED INFORMATION

CLR  R0                CLEAR ERROR FLAG

MOV  @CDBBUF+4,R1      MSW ZERO ( <= 16-BIT SECTOR SIZE )  ?

JNE  ITGBSE            NO, BAD SECTOR SIZE

MOV  @CDBBUF+6,R1      LSW ZERO ( NO SECTOR SIZE ) ?

JEQ  ITGBSE            YES, RETURN ERROR

SRL  R1,4              CREATE LOOP COUNTER FOR DMA READ/WRITE

JMP  INTRGX            RETURN OK

* ERROR RETURNS

ITGTPE LI   R0,ERRTYP*256     BAD DEVICE TYPE FOR SECTOR R/W

JMP  INTRGX

ITGSLE LI   R0,ERRSEL*256     SELECTION ERROR

JMP  INTRGX

ITGRDE LI   R0,ERRRTO*256     READY TIMEOUT

JMP  INTRGX

ITGRCE LI   R0,ERRBST*256     BAD STATUS RETURNED FOR READ CAPACITY

JMP  INTRGX

ITGBSE LI   R0,ERRBSE*256     BLOCK SIZE ERROR

INTRGX MOV  R0,*R13           SET R0 IN OLD WS WITH ERROR FLAG

MOV  R1,@2(R13)        COPY BLOCK SIZE TO OLD WS R1

RTWP                   RETURN

* SCSI COMMANDS USED BY THIS ROUTINE

RCPCMD DATA >2500,>0000,>0000,>0000,>0000   READ CAPACITY

RSECMD DATA >0300,>0000,>0400               REQUEST SENSE

INQCMD DATA >1200,>0000,>0200               INQUIRY