*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*       Sector read/write subroutines ( >10 and >20 )       *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

* Filename : SCSISECT/S                                     *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

*------------------------------------------------------------

* NAME : SECRW

*

* DESC : READ/WRITE SINGLE SECTOR ROUTINE ( >10 AND >20 )

*

* PARM : PAD DSR AREA LOADED WITH >10/>20 INFORMATION

*

* RET  : ERROR CODES RETURNED IN >8350

*

* DEST : R0,R1,R2,R3,R4,R5,R6,R7,R8,R10

DRVINS EQU  >D496             MOVB *R6,*R2

DRCINS EQU  >DC96             MOVB *R6,*R2+

DWVINS EQU  >D592             MOVB *R2,*R6

DWCINS EQU  >D5B2             MOVB *R2+,*R6

* ENTRY POINT FOR >10 SECTOR ROUTINE ( SHORT BLOCK ADDRESS )

SECRWS CLR  R9                Zero block MSW for >10 routine

JMP  SECRW

* ENTRY POINT FOR >20 SECTOR ROUTINE ( LONG BLOCK ADDRESS )

SECRWL MOV  @>8352,R9         Fetch MSW block for >20 routine

SECRW  MOV  R11,R10           Save return address

SECRWR MOVB @>834C,R8         Fetch unit # and flags

* IF SECTOR SIZE IS UNKNOWN THEN FIND IT

MOVB R8,R2             Unit number + flags

ANDI R2,SRWFMC*256     Mask out flags and LSB

SRL  R2,7              2 * (unit number)

MOV  @SECLTB(R2),R1    Do we know the sector size ?

JNE  SECW0A            Yes, now check the known flag

MOV  R2,R0             Copy (unit number) * 2

SRL  R0,1              Unit number

BLWP @INTERG           Interrogate SCSI device

MOV  R0,R0             Error finding size ?

JEQ  SECW0A            No, proceed

B    @SRWEXT           Yes

* ONLY ALLOW NON-256 BYTE SECTORS IF KNOWN FLAG SET

SECW0A MOV  R1,@SECLTB(R2)    Save sector size in table

MOV  R8,R0             No, copy unit number + flags

ANDI R0,SKNOWN*256     Sector size known flag set ?

JNE  SECW0B            Yes, proceed normally

CI   R1,16             No, so are they 256 byte sectors ?

JEQ  SECW0B            Yes, no problem

B    @SRWSSE           No, return error

* SELECT THE TARGET UNIT

SECW0B MOV  R8,R0             Copy unit number

SRL  R0,8              To LSB

ANDI R0,SRWFMC         Clear flag bits

BL   @SELECT           Attempt to select target

MOV  R0,R0             Success ?

JEQ  SECRW0            Yes, selection success

B    @SRWSLE           No, selection failed

* CONSTRUCT CDB FOR READ OR WRITE

SECRW0 LI   R0,CDBBUF         Pointer to CDB buffer in CPU RAM

MOV  R0,R2             Make a copy

LI   R1,>2800          SCSI Extended Read opcode

MOVB @>834D,R3         Read operation ?

JNE  SECRW1            Yes

LI   R1,>2A00          No, SCSI Extended Write opcode

SECRW1 MOV  R1,*R0+           Save opcode and LUN Bytes

MOV  R9,*R0+           Set MSW of block address

MOV  @>8350,*R0+       Set LSW of block address

CLR  *R0+              Clear PAD byte and count MSB

LI   R1,>0100          One block transfer + clear link

MOV  R1,*R0+           Save in CDB

* SEND THE CDB TO THE TARGET UNIT FROM CPU RAM

MOV  R2,R0             Pointer to CDB

LI   R1,10             Length of CDB in bytes

LI   R2,PHCMD          Command phase transfer

BL   @NWRITC           Write CDB from CPU RAM

CI   R0,10             All CDB bytes transferred ?

JEQ  SECW1A            Yes

B    @SRWCME           No, command error

* DATA TRANSFER PHASE... SAVE PAD AND CHECK FOR READ OR WRITE

SECW1A BL   @SAVPAD           Save PAD data for overlay

MOV  @>834E,R2         Data buffer address

MOVB @>834D,R1         Write operation ?

JEQ  SECW2A            Yes

* SET THE VDP WRITE ADDRESS IF DATA IS IN VDP RAM

LI   R4,PHDATI*256     Data in phase

LI   R6,RIDR           Input DMA data register

SLA  R8,1              Buffer in VDP RAM ?

JOC  SECRW2            No, buffer is in CPU RAM

MOV  R2,R0             Yes, copy buffer pointer

BL   @SETWDA           Set VDP write address

LI   R2,VDPWD          Write to VDP

LI   R3,DRVINS         Read-into-VDP-RAM instruction

JMP  SECRW4

SECRW2 LI   R3,DRCINS         Read-into-CPU-RAM instruction

JMP  SECRW4

* SET THE VDP READ ADDRESS IF DATA IS IN VDP RAM

SECW2A LI   R4,PHDATO*256     Data out phase

LI   R6,WODR           Output DMA data register

SLA  R8,1              Buffer in VDP RAM ?

JOC  SECRW3            No, buffer is in CPU RAM

MOV  R2,R0             Yes, copy buffer pointer

BL   @SETRDA           Set VDP read address

LI   R2,VDPRD          Read from VDP

LI   R3,DWVINS         Read-into-VDP-RAM instruction

JMP  SECRW4

SECRW3 LI   R3,DWCINS         Read-into-CPU-RAM instruction

* LOAD THE READ/WRITE CODE INTO THE PAD

SECRW4 LI   R0,PAD            PAD code buffer pointer

LI   R1,16             16 MOVB instructions

SECRW5 MOV  R3,*R0+           Copy instructions

DEC  R1

JNE  SECRW5

LI   R1,PDTAIL         Pointer to tail code

MOV  *R1+,*R0+         DEC R1

MOV  *R1+,*R0+         JNE $-36

MOV  *R1+,*R0+         B   *R11

* WAIT FOR DATA-IN, DATA-OUT OR STATUS PHASE

SECW5A MOVB @RCSB,R0          Current SCSI bus

ANDI R0,>1C00          Mask all but phase bits

JEQ  SECW5B            Jump if data out phase

CI   R0,PHSTAT*256*4   Status phase ?

JEQ  SECW10            Yes, abort the data phase

CI   R0,PHDATI*256*4   Data in phase ?

JNE  SECW5A            No.. keep waiting

* PERFORM THE SECTOR DMA READ/WRITE

SECW5B LI   R0,MR2DMA*256     DMA mode enable

MOVB R4,@WTCR          Set data in/out phase for transfer

JNE  SECRW6            Jump if data in phase

LI   R1,ICRBUS*256     Assert data bus for data out phase

MOVB R1,@WICR          Assert data bus

MOVB R0,@WMR2          Enable data DMA transfer

CLR  R0

MOVB R0,@WSDS          Start DMA write

JMP  SECRW7

SECRW6 MOVB R0,@WMR2          Enable data DMA transfer

CLR  R0

MOVB R0,@WSDI          Start DMA receive

SECRW7 SWPB R8                Unit + Known flag to LSB

ANDI R8,SRWFMC*2       Mask out Known flag

MOV  @SECLTB(R8),R1    Sector length from table

SBZ  PDMA              Enable P-DMA auto handshake

SECRW8 TB   DRQ               Wait for DRQ at start of block

JNE  SECRW8

X    R3                Read/Write first byte

SECRW9 TB   DRQ               Wait for DRQ after first byte

JNE  SECRW9            Necessary for writes !!

BL   @PAD+2            Fast PAD read code

SBO  PDMA              Disable P-DMA auto handshake

MOVB R1,@WMR2          Disable DMA transfer

MOVB R1,@WICR          Deassert data for writes

* RESTORE THE PAD TO INITIAL STATE

SECW10 BL   @RSTPAD           Restore PAD

* READ STATUS BYTE FROM TARGET

LI   R0,STSBUF         Status byte buffer

LI   R1,1              One byte to transfer

LI   R2,PHSTAT         Status phase

BL   @NREADC           Read status byte

DEC  R0                Get status byte ?

JNE  SRWSTE            No, didn't get status byte

* READ MESSAGE BYTE FROM TARGET

LI   R0,MSGBUF         Message byte buffer

LI   R1,1              One byte to transfer

LI   R2,PHMSGI         Message in phase

BL   @NREADC           Read message byte

DEC  R0                Get message byte ?

JNE  SRWMSE            No, didn't get message byte

* CHECK RETURNED SCSI STATUS BYTE FOR ERROR

MOVB @STSBUF,R0        Non-zero status byte ?

JEQ  SECW11            Yes, status error

* IF UNIT ATTENTION THEN NEED TO INTERROGATE UNIT AGAIN

MOVB @>834C,R2         FETCH UNIT NUMBER

SRL  R2,8              TO LSB

ANDI R2,SRWFMC         MASK OUT FLAG BITS

MOV  R2,R0             TO R0 FOR REQUEST SENSE ROUTINE

BLWP @RSENSE           PERFORM REQUEST SENSE

CI   R0,6              UNIT ATTENTION CONDITION ?

JNE  SRWBSE            NO, JUST RETURN STATUS ERROR

MOV  R2,R0             COPY UNIT NUMBER

BLWP @INTERG           FIND NEW BLOCK SIZE

MOV  R0,R0             ERROR ?

JNE  SRWBSE            YES, JUST RETURN WITH ERROR

SLA  R2,1              YES, MAKE UNIT INTO TABLE OFFSET

MOV  @SECLTB(R2),R0    FETCH OLD SECTOR SIZE

MOV  R1,@SECLTB(R2)    SAVE NEW SECTOR SIZE

C    R0,R1             SAME SIZE SECTORS ON NEW MEDIA ?

JNE  SRWCSE            NO, SECTOR SIZE HAS CHANGED.. ERROR

B    @SECRWR           RE-INTERROGATE AND RETRY

* NO ERRORS.. RETURN WITH CLEAR ERROR BYTE

SECW11 CLR  R0                No, return with zero error code

JMP  SRWEXT            Exit routine

* ERROR RETURNS WITH ERROR CODES

SRWCSE LI   R0,ERRSSC*256     Sector size changed

JMP  SRWEXT

SRWSSE LI   R0,ERRBSE*256     Sector size error

JMP  SRWEXT

SRWSLE LI   R0,ERRSEL*256     Selection failed

JMP  SRWEXT

SRWCME LI   R0,ERRCMD*256     Command error

JMP  SRWEXT

SRWSTE LI   R0,ERRSTS*256     No status byte

JMP  SRWEXT

SRWMSE LI   R0,ERRMSG*256     No message byte

JMP  SRWEXT

SRWBSE LI   R0,ERRBST*256     Bad status code returned

* RETURN TO CALLER

SRWEXT MOVB R0,@>8350         Save error code

MOV  @>8350,@>834A

B    *R10              Return

* TAIL CODE APPENDED TO PAD MOVB CODE

PDTAIL DEC  R1

JNE  PDTAIL-32

B    *R11

*------------------------------------------------------------

* NAME : RSENSE

*

* DESC : ISSUE REQUEST SENSE COMMAND AND RETURN SENSE KEY

*

* PARM : R0 = TARGET SCSI ADDRESS

*

* RET  : R0 = SENSE KEY IN LSB

*           = >FFFF IF ERROR GETTING SENSE INFORMATION

*

* DEST : R0,R1,R2,R3,R4,R5,R6,R7,R8,R10

RSENSE DATA LLWS1,RSENS1      BLWP VECTOR FOR ROUTINE

RSENS1 MOV  *R13,R0           FETCH R0 FROM OLD WS

BL   @SELECT           SELECT TARGET UNIT

MOV  R0,R0             ERROR SELECTING ?

JNE  RSENER            YES, RETURN ERROR

LI   R0,RSECMD         POINTER TO REQUEST SENSE CDB

LI   R1,6              6 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           ISSUE COMMAND TO TARGET UNIT

CI   R0,6              6 COMMAND BYTES TRANSFERRED ?

JNE  RSENER            NO, RETURN ERROR

LI   R0,CDBBUF         POINTER TO BUFFER FOR 4 SENSE BYTES

LI   R1,4              GET 4 SENSE BYTES

LI   R2,PHDATI         DATA IN PHASE

BL   @NREADC           READ SENSE DATA

LI   R0,STSBUF         BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,MSGBUF         BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

MOVB @CDBBUF+2,R0      FETCH SENSE KEY

SRL  R0,8              TO LSB

MOV  R0,*R13           STORE IN R0 OF OLD WS

RTWP                   RETURN

RSENER SETO *R13              SET ERROR FLAG IN R0 OF OLD WS

RTWP                   RETURN

*------------------------------------------------------------

* NAME : SAVPAD, RSTPAD

*

* DESC : SAVE AND RESTORE PAD STATE ROUTINES

*

* PARM : NONE

*

* RET  : NONE

*

* DEST : R0,R1,R2

SAVSIZ EQU  19                Number of PAD words to buffer

SAVPAD LI   R0,PAD            PAD is source

LI   R1,PADBUF         Buffer is destination

JMP  SRPAD1

RSTPAD LI   R0,PADBUF         Buffer is source

LI   R1,PAD            PAD is destination

SRPAD1 LI   R2,SAVSIZ         Number of words to save

SRPAD2 MOV  *R0+,*R1+         Copy the words

DEC  R2

JNE  SRPAD2

B    *R11              Return