*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*           Test code for low level SCSI routines           *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

* REFERENCES TO EXTERNAL SYMBOLS FROM LOW LEVEL LIBRARY

REF  PWRUP

REF  INTRPT

REF  INTERG

REF  SECRWS

REF  SECRWL

REF  FORMAT

REF  SCSICP

REF  EXAMIN

* DEFINITION FOR ENTRY POINT

DEF  START

COPY "DSK5.SCSIEQU/S"

* START OF TEST CODE

START  BLWP @TEST

B    *R11

TEST   DATA  MYWSP,TEST1

TEST1  LI   R12,>1000

BL   @PWRUP

LI   R0,5

BLWP @INTERG

*      LI   R0,>0503

*      MOV  R0,@>834C

*      LI   R0,VALSTR

*      MOV  R0,@>834E

*      BL   @FORMAT

*      LI   R0,>8500

*      MOV  R0,@>834C

*      LI   R0,>2000

*      MOV  R0,@>834E

*      BL   @EXAMIN

CLR  R0

LI   R1,>D000

LI   R2,>400

FILL   MOVB R0,*R1+

AI   R0,>100

DEC  R2

JNE  FILL

LI   R0,WDATA

LI   R1,PBASE

LI   R2,6

LOADI1 MOV  *R0+,*R1+

DEC  R2

JNE  LOADI1

BL   @SECRWL

LI   R0,RDATA

LI   R1,PBASE

LI   R2,6

LOADI2 MOV  *R0+,*R1+

DEC  R2

JNE  LOADI2

BL   @SECRWL

LI   R0,>2000

SWPB R0

MOVB R0,@VDPWA

SWPB R0

MOVB R0,@VDPWA

LI   R0,>D400

LI   R1,>400

LOOP   MOVB @VDPRD,*R0+

DEC  R1

JNE  LOOP

SBZ  0

RTWP

RDATA  DATA 0,>45FF,>2000,0,0,0

WDATA  DATA 0,>C500,>D000,0,0,0

MYWSP  BSS  32

EVEN

END