*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*              SCSI device examination routine              *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

*------------------------------------------------------------

* NAME : EXAMIN

*

* DESC : FETCHES INQUIRY AND CAPACITY DATA

*

* PARM : >834C = UNIT NUMBER + FLAGS

*        >834E = CPU BUFFER POINTER FOR INFORMATION

*

* RET  : >8350 = ERROR CODE OR ZERO IF OK

*

* DEST : R0,R1,R2,R3,R4,R5,R6,R7,R10

* ISSUE A SCSI INQUIRY COMMAND AND EXTRACT INFORMATION

EXAMIN MOV  R11,R10           SAVE RETURN ADDRESS

MOVB @>834C,R0         FETCH UNIT NUMBER

SLA  R0,1              KILL MS BIT

SRL  R0,9              TO LSB

BL   @SELECT           SELECT TARGET UNIT

MOV  R0,R0             SELECT SUCCESSFUL ?

JNE  EXMSLE            NO, SELECTION FAILED

LI   R0,IQCMD          POINTER TO INQUIRY COMMAND

LI   R1,6              6 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           SEND COMMAND TO TARGET

CI   R0,6              ALL CDB BYTES SENT ?

JNE  EXMCME            NO, TRY AGAIN

MOV  @>834E,R0         BUFFER FOR INQUIRY DATA

LI   R1,>24            36 BYTES OF INQUIRY DATA TO READ

LI   R2,PHDATI         DATA IN PHASE

MOVB @>834C,R3         CPU RAM BUFFER ?

JLT  EXAM1             NO, USE VDP RAM

BL   @NREADC           READ INQUIRY DATA INTO CPU RAM

JMP  EXAM2

EXAM1  BL   @NREADV           READ INQUIRY DATA INTO VDP RAM

EXAM2  LI   R0,STSBUF         BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,MSGBUF         BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

MOVB @STSBUF,R0        BAD STATUS ?

JNE  EXMBSE            YES, RETURN ERROR

* ISSUE A READ CAPACITY COMMAND TO DETERMINE DEVICE AND BLOCK SIZE

MOV  @>834C,R0         FETCH UNIT NUMBER

SLA  R0,1              KILL MS BIT

SRL  R0,9              TO LSB

BL   @SELECT           SELECT TARGET UNIT

MOV  R0,R0             SELECT SUCCESSFUL ?

JNE  EXMSLE            NO, SELECTION FAILED

LI   R0,RCCMD          POINTER TO READ CAPACITY COMMAND

LI   R1,10             10 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           SEND COMMAND TO TARGET

CI   R0,10             ALL CDB BYTES SENT ?

JNE  EXMCME            NO, TRY AGAIN

MOV  @>834E,R0         FETCH INFORMATION BUFFER POINTER

AI   R0,>24            CAPACITY DATA AFTER INQUIRY DATA

LI   R1,8              8 BYTES OF CAPACITY DATA TO READ

LI   R2,PHDATI         DATA IN PHASE

MOVB @>834C,R3         CPU RAM BUFFER ?

JLT  EXAM3             NO, USE VDP RAM

BL   @NREADC           READ CAPACITY DATA INTO CPU RAM

JMP  EXAM4

EXAM3  BL   @NREADV           READ CAPACITY DATA INTO VDP RAM

EXAM4  LI   R0,STSBUF         BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,MSGBUF         BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

CLR  R0                CLEAR ERROR FLAG

JMP  EXMEXT            RETURN WITH NO ERRORS

* ERROR RETURNS

EXMSLE LI   R0,ERRSEL*256     SELECTION ERROR

JMP  EXMEXT

EXMCME LI   R0,ERRCMD*256     READY TIMEOUT

JMP  EXMEXT

EXMBSE LI   R0,ERRBST*256     BAD STATUS RETURNED FOR READ CAPACITY

EXMEXT MOVB R0,@>8350         SET ERROR CODE

B    *R10              RETURN

* SCSI COMMANDS USED BY THIS ROUTINE

RCCMD  DATA >2500,>0000,>0000,>0000,>0000   READ CAPACITY

IQCMD  DATA >1200,>0000,>2400               INQUIRY