*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*             SCSI device formatting subprogram             *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

*------------------------------------------------------------

* NAME : FORMAT

*

* DESC : FORMAT A SCSI DEVICE TO A DEFINED SECTOR SIZE

*

* PARM : >834C = UNIT NUMBER

*        >834D = SECTOR SIZE BYTE

*        >834E = POINTER TO "FORMAT" VALIDATION STRING

*

* RET  : >8350 = ERROR CODES

*

* DEST : R0,R1,R2,R3,R4,R5,R6,R7,R8,R9,R10

* CHECK THE VALIDATION STRING BEFORE PROCEEDING

FORMAT MOV  R11,R10           SAVE RETURN ADDRESS

LI   R0,VALSTR         POINTER TO "FORMAT" STRING

MOV  @>834E,R1         VALIDATION STRING POINTER

LI   R2,6              6 BYTES TO COMPARE

FMT1   CB   *R0+,*R1+         MATCH CHARACTER ?

JNE  FMTVLE            NO, ABORT FORMATTING

DEC  R2                COMPARED 6 BYTES ?

JNE  FMT1              NO, KEEP GOING

* CALCULATE THE SECTOR SIZE

MOVB @>834D,R0         FETCH SECTOR SIZE BYTE

SRL  R0,8              TO LSB

LI   R1,128            SMALLEST SECTOR SIZE

ANDI R0,7              ZERO ( 128 BYTE SECTORS ) ?

JEQ  FMT2              YES

SLA  R1,0              NO, SHIFT TO GET SECTOR SIZE

* CONSTRUCT THE MODE SELECT PARAMETER DATA

FMT2   LI   R0,MSLBUF         POINTER TO MODE SELECT BUFFER

MOV  R2,*R0+           CLEAR MSW OF TRANSFER COUNT

LI   R3,8              8 MODE SELECT BYTES

MOV  R3,*R0+           SET LSW OF TRANSFER COUNT

MOV  R2,*R0+           CLEAR MSW OF DATA BLOCK COUNT

MOV  R2,*R0+           CLEAR LSW OF DATA BLOCK COUNT

MOV  R2,*R0+           CLEAR MSW OF BLOCK SIZE SPECIFIER

MOV  R1,*R0            SET LSW OF BLOCK SIZE SPECIFIER

* ISSUE A MODE SELECT COMMAND TO SET SECTOR SIZE

MOVB @>834C,R0         FETCH UNIT NUMBER

SRL  R0,8              TO LSB

BL   @SELECT           ATTEMPT TO SELECT TARGET UNIT

MOV  R0,R0             SELECT SUCCESSFUL ?

JNE  FMTSLE            NO, SELECTION FAILED

LI   R0,MSLCMD         POINTER TO MODE SELECT COMMAND

LI   R1,6              6 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           SEND COMMAND TO TARGET

CI   R0,6              ALL CDB BYTES SENT ?

JNE  FMTCDE            NO, ERROR

LI   R0,MSLBUF         POINTER TO MODE SELECT PARAMETERS

LI   R1,12             12 BYTES OF PARAMETERS TO SEND

LI   R2,PHDATO         DATA OUT PHASE

BL   @NWRITC           WRITE MODE SELECT PARAMETERS

LI   R0,>834A          BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,>834B          BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

MOVB @>834A,R0         BAD STATUS ?

JNE  FMTBSE            YES, RETURN ERROR

* ISSUE A FORMAT COMMAND TO THE TARGET UNIT

MOVB @>834C,R0         FETCH UNIT NUMBER

SRL  R0,8              TO LSB

BL   @SELECT           ATTEMPT TO SELECT TARGET UNIT

MOV  R0,R0             SELECT SUCCESSFUL ?

JNE  FMTSLE            NO, SELECTION FAILED

LI   R0,FMTCMD         POINTER TO MODE SELECT COMMAND

LI   R1,6              6 BYTE COMMAND

LI   R2,PHCMD          COMMAND PHASE

BL   @NWRITC           SEND COMMAND TO TARGET

CI   R0,6              ALL CDB BYTES SENT ?

JNE  FMTCDE            NO, ERROR

LI   R0,>834A          BYTE BUFFER FOR STATUS

LI   R1,1              1 STATUS BYTE

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE FROM TARGET

LI   R0,>834B          BYTE BUFFER FOR MESSAGE

LI   R1,1              1 MESSAGE BYTE

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE FROM TARGET

MOVB @>834A,R0         BAD STATUS ?

JEQ  FMTEXT            NO

* ERROR RETURNS

FMTBSE LI   R0,ERRBST*256     BAD STATUS RETURNED

JMP  FMTEXT

FMTCDE LI   R0,ERRCMD*256     BAD DEVICE TYPE FOR SECTOR R/W

JMP  FMTEXT

FMTSLE LI   R0,ERRSEL*256     SELECTION ERROR

JMP  FMTEXT

FMTVLE LI   R0,ERRVAL*256     FORMAT VALIDATION STRING ERROR

FMTEXT MOVB R0,@>8350         SET ERROR CODE IF ANY

B    *R10              RETURN

* SCSI COMMANDS AND DATA USED BY THIS ROUTINE

MSLCMD DATA >1501,>0000,>0C00   MODE SELECT

FMTCMD DATA >0400,>0000,>0000   FORMAT UNIT

VALSTR TEXT 'FORMAT'            VALIDATION STRING

EVEN