*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*   Subroutine for SCSI-Direct communication with devices   *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

*------------------------------------------------------------

* NAME : SCSICP

*

* DESC : ISSUE A GENERIC SCSI COMMAND TO A TARGET UNIT

*

* PARM : PAD DSR AREA LOADED WITH COMMAND STRUCTURE

*

* RET  : ERROR CODES RETURNED IN >8350

*

* DEST : R0,R1,R2,R3,R4,R5,R6,R7,R8,R10

* ATTEMPT TO SELECT THE TARGET UNIT

SCSICP MOV  R11,R10           SAVE RETURN ADDRESS

MOVB @SCUNIT,R8        FETCH UNIT # AND FLAGS

MOV  R8,R0             COPY UNIT NUMBER

SRL  R0,8              TO LSB

ANDI R0,SCUFMC         CLEAR FLAG BITS

BL   @SELECT           ATTEMPT TO SELECT TARGET

MOV  R0,R0             SUCCESS ?

JNE  SCSELE            NO, SELECTION FAILED

* SEND THE CDB TO THE TARGET UNIT FROM VDP OR CPU RAM

MOV  @SCCDBP,R0        POINTER TO CDB

MOV  @SCCDBL,R1        LENGTH OF CDB IN BYTES

LI   R2,PHCMD          COMMAND PHASE TRANSFER

MOV  R8,R8             CDB IN VDP RAM ?

JLT  SCSIC1            NO, CDB IS IN CPU RAM

BL   @NWRITV           YES, WRITE CDB FROM VDP RAM

JMP  SCSIC2

SCSIC1 BL   @NWRITC           WRITE CDB FROM CPU RAM

SCSIC2 C    @SCCDBL,R0        ALL CDB BYTES TRANSFERRED ?

JNE  SCCMDE            NO, COMMAND ERROR

* DATA TRANSFER PHASE

MOV  @SCDATP,R0        POINTER TO DATA BUFFER

MOV  @SCDATL,R1        LENGTH OF DATA TO BE TRANSFERRED

JEQ  SCSIC7            NO DATA PHASE IF ZERO LENGTH

MOVB @SCFLAG,R3        READ OPERATION ?

JNE  SCSIC4            YES, READ

* WRITE DATA TO TARGET FROM VDP OR CPU RAM

LI   R2,PHDATO         NO, USE DATA OUT PHASE FOR WRITE

SLA  R8,2              DATA IN VDP RAM ?

JOC  SCSIC3            YES, USE CPU RAM WRITE

BL   @NWRITV           NO, WRITE DATA FROM VDP RAM

JMP  SCSIC6

SCSIC3 BL   @NWRITC           WRITE DATA FROM CPU RAM

JMP  SCSIC6

* READ DATA FROM TARGET TO VDP OR CPU RAM

SCSIC4 LI   R2,PHDATI         USE DATA IN PHASE FOR READ

SLA  R8,2              DATA IN VDP RAM ?

JOC  SCSIC5            YES, USE CPU RAM READ

BL   @NREADV           READ DATA INTO VDP RAM

JMP  SCSIC6

SCSIC5 BL   @NREADC           READ DATA INTO CPU RAM

SCSIC6 MOV  R0,@SCDATL        SAVE ACTUAL LENGTH TRANSFERRED

* MAKE SURE WE ARE IN STATUS PHASE

SCSIC7 MOVB @RCSB,R0               FETCH SCSI BUS SIGNALS

ANDI R0,CSBMSG+CSBCD+CSBIO  MASK OUT ALL BUT PHASE BITS

CI   R0,PHSTAT*256*4        IN STATUS PHASE ?

JNE  SCPHSE                 NO, PHASE ERROR

* READ STATUS BYTE FROM TARGET

LI   R0,SCSTAT         BYTE BUFFER IN PAD

LI   R1,1              ONE BYTE TO TRANSFER

LI   R2,PHSTAT         STATUS PHASE

BL   @NREADC           READ STATUS BYTE

DEC  R0                GET STATUS BYTE ?

JNE  SCBADS            NO, DIDN'T GET STATUS BYTE

* READ MESSAGE BYTE FROM TARGET

LI   R0,SCMESS         BYTE BUFFER IN PAD

LI   R1,1              ONE BYTE TO TRANSFER

LI   R2,PHMSGI         MESSAGE IN PHASE

BL   @NREADC           READ MESSAGE BYTE

DEC  R0                GET MESSAGE BYTE ?

JNE  SCBADM            NO, DIDN'T GET MESSAGE BYTE

* CHECK RETURNED SCSI STATUS BYTE FOR ERROR

MOVB @SCSTAT,R0        NON-ZERO STATUS BYTE ?

JNE  SCSTSE            YES, STATUS ERROR

CLR  R0                NO, RETURN WITH ZERO ERROR CODE

JMP  SCEXIT            EXIT ROUTINE

* ERROR RETURNS WITH ERROR CODES

SCSELE LI   R0,ERRSEL*256     SELECTION FAILED

JMP  SCEXIT

SCCMDE LI   R0,ERRCMD*256     COMMAND ERROR

JMP  SCEXIT

SCBADS LI   R0,ERRSTS*256     NO STATUS BYTE

JMP  SCEXIT

SCBADM LI   R0,ERRMSG*256     NO MESSAGE BYTE

JMP  SCEXIT

SCPHSE LI   R0,ERRPHS*256     PHASE ERROR

JMP  SCEXIT

SCSTSE LI   R0,ERRBST*256     BAD STATUS CODE RETURNED

* RETURN TO CALLER

SCEXIT MOVB R0,@>8350         SAVE ERROR CODE

B    *R10              RETURN