*---------------------------------------------------------------*

|  Programmer Documentation for WHT-SCSI Low Level Subprograms  |

|                                                               |

|                      BETA RELEASE 1                           |

|                                                               |

|                 Written by Will McGovern                      |

*---------------------------------------------------------------*

Use of the Dis/Fix 80 ( linkable ) low level library

----------------------------------------------------

All subprograms behave as documented below.  The corresponding

DEF'd symbols in the library are as follows :

PWRUP         LOW LEVEL POWER UP ROUTINE ( MUST BE CALLED )

INTRPT (BLWP) INTERRUPT HANDLER ROUTINE

INTERG        WAIT FOR UNIT READY / INTERROGATE SECTOR SIZE

SECRWS        SECTOR READ/WRITE     ( >10 )

SECRWL        SECTOR READ/WRITE     ( >20 )

FORMAT        SCSI LOW LEVEL FORMAT ( >21 )

EXAMIN        EXAMINE SCSI DEVICE   ( >18 )

SCSICP        SCSI DIRECT COMMAND   ( >17 )

All routines are entered using BL @<name> with parameters as documented.

These are written to be incorporated directly into the final DSR.

When using the Dis/Fix 80 linkable code you must manually call the

low level powerup routine before any others as follows :

LI   R12,>1x00     CRU base address of SCSI card

BL   @PWRUP        Initialise 58c80 and SCSI bus

The interrupt routine should not be necessary, but will clear a

pending SCSI interrupt.  If the system locks up with all the PE

expansion card LED's flashing it means that there is a hanging

SCSI interrupt and the console software is searching for a DSR

to clear it.  Use the INTRPT routine to clear this interrupt.

Note that these interrupts should NOT occur unless something

went badly wrong with a data transfer.

The INTERG routine is provided to allow waiting for drive ready

after the initial SCSI reset ( generated by PWRUP ).  This

routine takes a unit number ( 0->7 ) in R0 as argument.  It

returns with R0 zero if OK ( non-zero = error ) and with the

device sector size in R1.  This routine must be called when

talking with a SCSI device for the first time ( except for

sector read/write ) after the power up routine has been called.

------------------------------------------------------------------------

EXAMPLE PROGRAM TO READ SECTOR ZERO FROM SCSI DRIVE AT ADDRESS 5

----------------------------------------------------------------

* References to external symbols from low level library

REF  PWRUP

REF  INTRPT

REF  INTERG

REF  SECRWS

REF  SECRWL

REF  FORMAT

REF  SCSICP

REF  EXAMIN

* DEFINITION FOR ENTRY POINT

DEF  START

* EQUATES FOR TI

VDPWA  EQU  >8C02

VDPRD  EQU  >8800

START  MOV  R11,@SAVRTN       Save return address

LI   R12,>1000         CRU base of my SCSI card

BL   @PWRUP            Call power up routine

LI   R0,5              My Quantum drive is at address 5

BLWP @INTERG           Wait for unit ready

* Find the sector size of the device

LI   R0,>8500          SCSI address 5 and CPU RAM buffer

MOVB R0,@>834C         Set unit number in PAD

LI   R0,INFO           Pointer to my information buffer

MOV  R0,@>834E         Save pointer in PAD

BL   @EXAMIN           Examine SCSI Device ( >18 )

MOVB @>8350,R0         Any errors ?

JNE  ERROR             Jump if error

MOV  @INFO+42,R0       Get sector size from buffer

CI   R0,512            512 byte sectors ?

JNE  ERROR             No, we only handle 512 byte sectors

* Now perform the sector read using the >20 subprogram

LI   R0,>C5FF          CPU RAM + KNOWN + Read data

MOV  R0,@>834C

LI   R0,SECBUF         Buffer for sector read

MOV  R0,@>834E

CLR  R0

MOV  R0,@>8350         LSW of 32-bit sector number 0

MOV  R0,@>8352         MSW of 32-bit sector number 0

BL   @SECRWL           Call sector read/write ( >20 )

MOVB @>8350,R0         Any errors ?

JNE  ERROR             Jump if error

* The sector has been read into the buffer now !!

ERROR  SBZ  0                 Turn off SCSI DSR ROM

MOV  @SAVRTN,R11       Fetch return address

B    *R11              Return

* Data buffers

SAVRTN BSS  2                 Return address storage

INFO   BSS  44                Info buffer for >18 subprogram

SECBUF BSS  512               Sector buffer for IBM size sectors

END

--------------------------------------------------------------------------------

Subprogram  : >10 and >20

Purpose     : Sector read/write

Description : Performs a single sector read/write on the specified SCSI

device.  Extensions have been added to permit operation

with sector sizes other than 256 bytes and to allow 32-bit

sector number specifications for large devices.  These

operations are only supported on direct access ( type 0 )

abd CDROM ( type 5 ) devices.

This subprogram has built-in intelligence to automatically

handle changes in block size inherent in the use of SCSI

floppy drives and other removable media devices.  To handle

the problem of media changes between media of different

sector sizes, an error reporting scheme has been devised.

This will only affect SCSI-aware programs.  When the DSR

detects that a new disk has been inserted which has a

different sector size to the last disk, a SECTOR SIZE

CHANGE error will be reported to any SCSI-aware program

on the FIRST attempted read/write operation.  Upon receipt

of this error code, the SCSI-aware program MUST call the

>18 Examine SCSI Unit subprogram to find the new sector

size.  The error code will only be returned on the FIRST

access to this subprogram.

Warning : Ignoring the SECTOR SIZE CHANGE error code will

cause MAJOR problems as the DSR will adapt to

the new sector size transparently, while your

program will still be expecting sectors of the

old size.

Note    : This special handling is only required if you

use the SECTOR SIZE KNOWN flag bit.  If this

bit remains clear, any sector size other than

256 bytes is considered an error.

Parameters  : >834A WORD  Returned sector number ( >10 only )

>834C BYTE  Unit number + flags

>834D BYTE  Read/write flag

>834E WORD  Buffer start address

>8350 WORD  Sector number

>8352 WORD  MSW of sector number ( >20 only )

UNIT NUMBER : CKxxxTTT

( >834C )     ||\_/\_/

|| |  |

|| |  *-- SCSI TARGET ADDRESS ( 0 -> 7 )

|| |

|| *----- RESERVED ( must be 0 )

||

|*------- SECTOR SIZE KNOWN FLAG

|

|         0 : Default for programs unable to handle sector

|             sizes other than 256 bytes.  An error is

|             returned if the specified SCSI target device

|             ( identified by target address bits TTT )

|             has a sector size other than 256 bytes.

|             This default allows for transparent operation

|             with non-SCSI-aware programs and prevents

|             them from blundering into devices with sector

|             sizes ( ie. not 256 ) they cannot handle.

|

|         1 : When set, this bit informs the DSR that the

|             caller is aware of the sector size of the

|             SCSI device ( from the >18 Inquire Sector

|             Size DSR subprogram ).  The entire sector,

|             regardless of its size, will be read into

|             the specified buffer.  This bit should only

|             be set by SCSI-aware programs that are able

|             to handle different sector sizes.

|

*-------- CPU RAM BUFFER FLAG

0 : When clear, indicates that the specifed buffer

is in VDP RAM.

1 : When set, indicates the specified buffer is

in CPU RAM.

READ/WRITE  : If zero, indicates a write operation to SCSI device.

( >834D )     If non-zero, indicates a read operation from SCSI device.

BUFFER      : Pointer to the start of the sector buffer for this operation.

( >834E )     The location of the buffer ( VDP or CPU RAM ) is governed

by the state of the MS-bit in the unit number.

SECTOR NUM  : For the >10 ( floppy ) subprogram, this is the 16-bit sector

( >8350 )     number for the operation.  The sector number is returned in

the location >834A upon completion of the operation.

For the >20 ( hard ) subprogram, this 16-bit word forms the

LSW of a 32-bit sector number.  The MSW of the sector number

MUST be defined in >8352.

MSW SECTOR  : Only valid for >20 subprogram.  Specifies the MSW of the

( >8352 )     32-bit sector number.

ERRORS      : Errors are returned in location >8350 ( byte ).

--------------------------------------------------------------------------------

--------------------------------------------------------------------------------

Subprogram  : >21

Purpose     : Format SCSI device

Description : Low level formats a SCSI device to a specified block

size.  For safety, the formatting will only proceed

if a verification string is provided.

Parameters  : >834C BYTE  Unit number + flags

>834D BYTE  Sector size ( see below for encoding )

>834E WORD  Pointer to validation string

UNIT NUMBER : xxxxxTTT

( >834C )     \___/\_/

|   |

|   *-- SCSI TARGET ADDRESS ( 0 -> 7 )

|

*------ RESERVED ( must be 0 )

SECTOR SIZE : xxxxxSSS

( >834D )     \___/\_/

|   |

|   *-- SECTOR SIZE SPECIFIER ( 0 -> 7 )

|

|       000 = 128   bytes/sector

|       001 = 256   bytes/sector

|       010 = 512   bytes/sector

|       011 = 1024  bytes/sector

|       100 = 2048  bytes/sector

|       101 = 4096  bytes/sector

|       110 = 8192  bytes/sector

|       111 = 16384 bytes/sector

|

|       Note that many drives only support a small subset

|       subset of these sizes.  An error will be returned

|       if the unit does not support the specified sector

|       size.

|

|       eg. QUANTUM LP52S  : 512 / 1024 / 2048  only

|           FUJITSU 261xSA : 256 / 512  / 1024  only

|

*------ RESERVED ( must be 0 )

VALIDATION  : This is a safety precaution to prevent accidental

( >834E )     formatting of a SCSI device.  This location MUST

contain a pointer to a 6 byte string in CPU RAM.

These 6 bytes must form the string "FORMAT" or the

format will be aborted with an error.

ERRORS      : Errors are returned in location >8350 ( byte ).

--------------------------------------------------------------------------------

--------------------------------------------------------------------------------

Subprogram  : >17

Purpose     : Direct SCSI Command

Description : This subprogram allows external programs to communicate directly

with SCSI devices at SCSI command level, without any knowledge

of the underlying SCSI controller hardware.  This provides a

simple and powerful interface to the SCSI bus for SCSI-aware

programs that wish to talk directly to SCSI devices.  This

subprogram requires a SCSI Command Descriptor Block ( CDB )

to issue to the SCSI device and a data buffer ( for those SCSI

commands involving data transfer ).

This subprogram uses programmed I/O and is intended for use

with SCSI commands NOT requiring large amounts of data to be

transferred during the data phase ( such as directly invoking

a read or write ).  In future revisions an optimized, block

oriented alternative may be provided for maintaining high

data throughput for sequential devices.

Parameters  : >834A BYTE  Returned SCSI status byte

>834B BYTE  Returned SCSI message byte

>834C BYTE  Unit number + flags

>834D BYTE  Read/write flag ( for data phase if any )

>834E WORD  Command Descriptor Block start address

>8350 WORD  Command Descriptor Block length in bytes

>8352 WORD  Data buffer start address

>8354 WORD  Maximum expected length of data in bytes

STATUS      : Status byte returned by SCSI unit on command completion.

( >834A )     Zero implies all went OK.  See ANSI SCSI spec for other

status codes.

MESSAGE     : Message byte returned by SCSI unit on command completion.

( >834B )     Usually nothing relevant.  See ANSI SCSI spec for other

message codes.

UNIT NUMBER : CDxxxTTT

( >834C )     ||\_/\_/

|| |  |

|| |  *-- SCSI TARGET ADDRESS ( 0 -> 7 )

|| |

|| *----- RESERVED ( must be 0 )

||

|*------- DATA MEMORY TYPE FLAG

|

|         0 : Data buffer for SCSI command is in VDP RAM.

|

|         1 : Data buffer for SCSI command in in CPU RAM.

|

*-------- COMMAND DESCRIPTOR BLOCK MEMORY TYPE FLAG

0 : Command descriptor block is in VDP RAM.

1 : Command descriptor block is in CPU RAM.

READ/WRITE  : If zero, indicates data phase will be out ( write ).

( >834D )     If non-zero indicates data phase will be in ( read ).

CDB BUFFER  : Pointer to the start of the buffer containing the SCSI

( >834E )     Command Descriptor Block.  The location of the buffer

( VDP or CPU RAM ) is governed by the state of bit 0 in

the unit field.  See ANSI SCSI spec for more information.

CDB LENGTH  : Length of the SCSI Command Descriptor Block in bytes.

( >8350 )     Usually 6, 10 or 12 bytes.  The actual number of command

bytes transferred is returned in this location.

DATA BUFFER : Pointer to the start of the data buffer for the data phase

( >8350 )     of the SCSI command.  The location of this buffer ( VDP or

CPU RAM ) is governed by bit 1 in the unit field.  This

pointer is NOT used if there is no data phase for the

command ( indicated by a zero data length ).

DATA LENGTH : Expected length of data to be transferred in data phase.

( >8352 )     Zero implies that the SCSI command has no data phase.

This is a maximum ( should be set to the maximum

size of your buffer ) and may not necessarily reflect

the number of bytes actually transferred.  The actual

number of data bytes transferred is returned in this

location.  If you do not specify a length large enough

to get the target unit to status phase ( ie. out of the

data phase ) then a phase error will result.

ERRORS      : Errors are returned in location >8350 ( byte ).

--------------------------------------------------------------------------------

--------------------------------------------------------------------------------

Subprogram  : >18

Purpose     : Examine SCSI Unit

Description : This subprogram interrogates a SCSI device and returns a packet

of relevant information to the caller.  The packet contains

device manufacturer and model identification, block size,

total number of blocks on device and device type.  This makes

life easier for external programs.

Parameters  : >834C BYTE  Unit number

>834E WORD  Pointer to buffer for information packet

UNIT NUMBER : CxxxxTTT

( >834C )     |\__/\_/

|  |  |

|  |  *-- SCSI TARGET ADDRESS ( 0 -> 7 )

|  |

|  *----- RESERVED ( must be 0 )

|

*-------- CPU RAM BUFFER FLAG

0 : When clear, indicates that the specifed buffer

is in VDP RAM.

1 : When set, indicates the specified buffer is

in CPU RAM.

BUFFER      : The structure of the information packet is as follows :

( >834E )

Offset  Description

------  -----------

0      8 bytes of information identical to the first 8

bytes of the SCSI devices response to an INQUIRY

command.  See the ANSI SCSI spec for more info

on these bytes.

8      8 bytes of SCSI device vendor ID string

16     16 bytes of SCSI device product ID string

32     4 bytes of SCSI product revision string

36     32-bit total number of sectors on device

40     32-bit device sector size

The total packet size is 44 bytes and the location of the

buffer ( VDP or CPU RAM ) is governed by bit 0 of the unit

byte.

Note : Some devices ( such as tape drives ) will not return

values for total number of sectors or sector size.

It is a good idea to check the device type in the

SCSI information to see whether the capacity info

is relevant to the device.

ERRORS      : Errors are returned in location >8350 ( byte ).

--------------------------------------------------------------------------------

ERROR CODES RETURNED BY LOW LEVEL SUBPROGRAMS :

>01 = Selection of target failed

>02 = SCSI command issue failure

>03 = No SCSI status phase detected

>04 = No SCSI message phase detected

>05 = Bad SCSI status returned by target

>06 = Ready timeout ( drive not ready )

>07 = Sector size error

>08 = Device type error

>09 = Bad format validation string

>0A = SCSI phase error

>0B = Sector size changed ( removable devices )

-----------------------------------------------------------------------

END OF DOCUMENT