*-----------------------------------------------------------*

*     Western Horizon Technologies 5380 SCSI Controller     *

*                                                           *

*        Equates for low level SCSI code for NCR5380        *

*                                                           *

*        Written and Copyright 1993 by Will McGovern        *

*-----------------------------------------------------------*

* HISTORY                                                   *

* -------                                                   *

*                                                           *

* 10-Jan-93  WMC  Created this file                         *

*-----------------------------------------------------------*

* 5380/8490 REGISTER EQUATES ( READ )

RCSD   EQU  >5FE0             CURRENT SCSI DATA

RICR   EQU  >5FE2             INITIATOR COMMAND

RMR2   EQU  >5FE4             MODE

RTCR   EQU  >5FE6             TARGET COMMAND

RCSB   EQU  >5FE8             CURRENT SCSI BUS STATUS

RBSR   EQU  >5FEA             BUS AND STATUS

RIDR   EQU  >5FEC             INPUT DATA REGISTER

RRPI   EQU  >5FEE             RESET PARITY/INTERRUPTS

RISR   EQU  >5FEE             INTERRUPT STATUS ( 8490 ONLY )

* 5380/8490 REGISTER EQUATES ( WRITE )

WODR   EQU  >5FF0             OUTPUT DATA

WICR   EQU  >5FF2             INITIATOR COMMAND

WMR2   EQU  >5FF4             TARGET COMMAND

WTCR   EQU  >5FF6             MODE

WSER   EQU  >5FF8             SELECT ENABLE

WSDS   EQU  >5FFA             START DMA SEND

WSDT   EQU  >5FFC             START DMA TARGET RECEIVE

WSDI   EQU  >5FFE             START DMA INITIATOR RECEIVE

WEMR   EQU  >5FFE             ENHANCED MODE ( 8490 ONLY )

WIMR   EQU  >5FFE             INTERRUPT MASK ( 8490 ONLY )

* PSEUDO-DMA DATA REGISTER ADDRESS

RDMA   EQU  >5FEC             READ DMA ADDRESS

WDMA   EQU  >5FF0             WRITE DMA ADDRESS

* CRU ASSIGNMENTS ( READ )

IRQ    EQU   0                INTERRUPT REQUEST

DRQ    EQU   1                DMA REQUEST

READY  EQU   2                READY

DIP0   EQU   4                DIP SWITCH 0

DIP1   EQU   5                DIP SWITCH 1

DIP2   EQU   6                DIP SWITCH 2

DIP3   EQU   7                DIP SWITCH 3

* CRU ASSIGNMENTS ( WRITE )

CARD   EQU   0                CARD ENABLE

SHADOW EQU   1                SRAM SHADOW

PDMA   EQU   2                PSEUDO DMA HANDSHAKE

EOP    EQU   3                END OF PROCESS

E13    EQU   8                EPROM A13

E14    EQU   9                EPROM A14

E15    EQU   10               EPROM A15

S12    EQU   12               SRAM A12

S13    EQU   13               SRAM A13

S14    EQU   14               SRAM A14

* BIT DEFINITIONS FOR ICR REGISTER

ICRRST EQU   >80              ASSERT RST*

ICRAIP EQU   >40              ARBITRATION IN PROGRESS

ICRMOD EQU   >40              OPERATING MODE ( 8490 ONLY )

ICRLA  EQU   >20              LOST ARBITRATION

ICRDIF EQU   >20              DIFFERENTIAL ENABLE

ICRACK EQU   >10              ASSERT ACK*

ICRBSY EQU   >08              ASSERT BSY*

ICRSEL EQU   >04              ASSERT SEL*

ICRATN EQU   >02              ASSERT ATN*

ICRBUS EQU   >01              ASSERT DATA BUS

* BIT DEFINITIONS FOR MODE REGISTER

MR2BLK EQU   >80              BLOCK MODE DMA

MR2TRG EQU   >40              TARGET/INITIATOR MODE

MR2PAR EQU   >20              PARITY CHECKING

MR2PI  EQU   >10              PARITY INTERRUPT

MR2EOP EQU   >08              EOP* INTERRUPT

MR2BSY EQU   >04              BSY* MONITOR

MR2DMA EQU   >02              DMA MODE

MR2ARB EQU   >01              ARBITRATE

* BIT DEFINITIONS FOR TARGET COMMAND REGISTER

TCRTED EQU   >80              TRUE END OF DMA ( 8490 ONLY )

TCRREQ EQU   >08              ASSERT REQ*

TCRMSG EQU   >04              ASSERT MSG*

TCRCD  EQU   >02              ASSERT C/D*

TCRIO  EQU   >01              ASSERT I/O*

* BIT DEFINITIONS FOR CURRENT SCSI BUS STATUS REGISTER

CSBRST EQU   >80              RST*

CSBBSY EQU   >40              BSY*

CSBREQ EQU   >20              REQ*

CSBMSG EQU   >10              MSG*

CSBCD  EQU   >08              C/D*

CSBIO  EQU   >04              I/O*

CSBSEL EQU   >02              SEL*

CSBDBP EQU   >01              DBP*

* BIT DEFINITIONS FOR BUS AND STATUS REGISTER

BSRED  EQU   >80              END OF DMA

BSRDRQ EQU   >40              DMA REQUEST

BSRSPR EQU   >20              SCSI PARITY ERROR

BSRINT EQU   >10              INTERRUPT REQUEST

BSRPHM EQU   >08              PHASE MATCH

BSRBSY EQU   >04              BUSY ERROR

BSRATN EQU   >02              ATN*

BSRACK EQU   >01              ACK*

* BIT DEFINITIONS FOR ENHANCED MODE REGISTER ( 8490 ONLY )

EMRAPM EQU   >80              ANY PHASE MISMATCH

EMRMPE EQU   >40              uP PARITY ENABLE

EMRMPO EQU   >20              uP PARITY POLARITY

EMRSPO EQU   >10              SCSI PARITY POLARITY

EMRLP  EQU   >08              LOOPBACK MODE

EMREF1 EQU   >04              EXTERNAL FUNCTION 1

EMREF0 EQU   >02              EXTERNAL FUNCTION 0

EMRARB EQU   >01              EXTENDED ARBITRATION

* BIT DEFINITIONS FOR INTERRUPT STATUS REGISTER ( 8490 ONLY )

ISRSPE EQU   >80              SCSI PARITY ERROR

ISRMPE EQU   >40              uP PARITY ERROR

ISRED  EQU   >20              END OF DMA

ISRDPS EQU   >10              DMA PHASE MISMATCH

ISRAPS EQU   >08              ANY PHASE MISMATCH

ISRBSY EQU   >04              BUSY LOSS

ISRSEL EQU   >02              SELECTION/RESELECTION

ISRARB EQU   >01              ARBITRATION COMPLETE

* TIMING EQUATES THAT MAY NEED ADJUSTING

RSTCNT EQU  200               RESET DELAY COUNT

ARBCNT EQU  1                 ARBITRATION DELAY COUNT 2.2us

BSCNT  EQU  2                 BUS SETTLE DELAY COUNT  600ns

BCCNT  EQU  2                 BUS CLEAR DELAY COUNT   600ns

BSYCNT EQU  200               BSY* WAIT DELAY COUNT   200ms

* SCSI PHASE EQUATES

PHDATO EQU  0                 DATA OUT

PHDATI EQU  1                 DATA IN

PHCMD  EQU  2                 COMMAND

PHSTAT EQU  3                 STATUS

PHMSGO EQU  6                 MESSAGE OUT

PHMSGI EQU  7                 MESSAGE IN

* ERROR CODES RETURNED BY SCSI ROUTINES

MINERR EQU  1                 STARTING ERROR CODE

ERRSEL EQU  MINERR            SELECTION FAILED

ERRCMD EQU  MINERR+1          SCSI COMMAND ERROR

ERRSTS EQU  MINERR+2          NO SCSI STATUS PHASE

ERRMSG EQU  MINERR+3          NO SCSI MESSAGE PHASE

ERRBST EQU  MINERR+4          BAD SCSI STATUS RETURNED

ERRRTO EQU  MINERR+5          READY TIMEOUT

ERRBSE EQU  MINERR+6          BLOCK SIZE ERROR

ERRTYP EQU  MINERR+7          DEVICE TYPE ERROR

ERRVAL EQU  MINERR+8          BAD VALIDATION STRING

ERRPHS EQU  MINERR+9          SCSI PHASE ERROR

ERRSSC EQU  MINERR+10         SECTOR SIZE CHANGED

* GENERAL TI SYSTEM EQUATES

VDPWA  EQU  >8C02             VDP WRITE ADDRESS

VDPRD  EQU  >8800             VDP READ DATA

VDPWD  EQU  >8C00             VDP WRITE DATA

PAD    EQU  >8300             CONSOLE CPU RAM ADDRESS

PBASE  EQU  >834A             DSR PARAMETER BASE ADDRESS

GPLWS  EQU  >83E0             GPL WORKSPACE

* EQUATES FOR THE SCSI-DIRECT COMMAND STRUCTURE

SCSTAT EQU  PBASE+0           STATUS RETURNED BY TARGET UNIT

SCMESS EQU  PBASE+1           MESSAGE RETURNED BY TARGET UNIT

SCUNIT EQU  PBASE+2           SCSI TARGET UNIT

SCFLAG EQU  PBASE+3           READ/WRITE FLAG

SCCDBP EQU  PBASE+4           CDB POINTER

SCCDBL EQU  PBASE+6           CDB LENGTH

SCDATP EQU  PBASE+8           DATA AREA POINTER

SCDATL EQU  PBASE+10          DATA LENGTH ( MAX = 65535 BYTES )

* FLAG BITS DEFINED FOR THE SCUNIT BYTE

CDBCPU EQU  >80               CDB IS IN CPU RAM

DATCPU EQU  >40               DATA BUFFER IS IN CPU RAM

SCUFM  EQU  >C0               MASK FOR ALL FLAG BITS

SCUFMC EQU  >3F               COMPLEMENT OF PREVIOUS

* FLAG BITS DEFINED FOR THE UNIT BYTE FOR SECTOR READ/WRITE

SECCPU EQU  >80               SECTOR DATA BUFFER IN CPU RAM

SKNOWN EQU  >40               SECTOR SIZE IS KNOWN

SRWFM  EQU  >C0               MASK FOR ALL FLAG BITS

SRWFMC EQU  >3F               COMPLEMENT OF PREVIOUS