*======================================================*

*                                                      *

* DIRECT ACCESS ROUTINES FOR THE MYARC DISK CONTROLLER *

*                                                      *

*======================================================*

*----------------------------------*

* Data for disk access subroutines *

*----------------------------------*

SMASK  EQU  >E3FF             Sector read/write error mask

FMASK  EQU  >DBFF             Write track error mask

TRKLEN EQU  >1900             Track length

SIDE   DATA >0000             Side 0/1 flag

T0     BYTE >0A               Restore to track zero command

ST     BYTE >1A               Seek track command

SI     BYTE >5A               Step head in command

RA     BYTE >C0

RS     BYTE >88               Read sector command

RT     BYTE >E4               Read address

WS     BYTE >A8               Write sector command

WT     BYTE >F0               Write track command

COMM   BYTE >00               Command byte

EVEN

SETPAD LI   R0,RWCD1

LI   R1,PAD

LI   R2,RWLGTH

SETPD1 MOVB *R0+,*R1+

DEC  R2

JNE  SETPD1

RT

*------------------------*

* Address read routine   *

*                        *

* BL   @TIORMS           *

*                        *

* Expects drive selected *

*------------------------*

ADDRES DATA 0,0,0

TIORMS MOVB @RA,@COMM

LI   R0,READ1

LI   R3,6

JMP  MOP

*------------------------*

* Sector read routine    *

*                        *

* BL   @MSECR            *

* JMP  ERR               *

*                        *

* Expects drive selected *

*------------------------*

MSECR  MOVB @RS,@COMM         Set command byte to read sector

LI   R0,READ1          Address of read sector code to move

JMP  MSECW1

*------------------------*

* Sector write routine   *

*                        *

* BL   @MSECW            *

* JMP  ERR               *

*                        *

* Expects drive selected *

*------------------------*

MSECW  MOVB @WS,@COMM         Set command byte to write sector

LI   R0,WRITE1         Address of write sector code to move

MSECW1 LI   R3,>200           Length of one MS-DOS sector (512 bytes)

MOVB R6,@>5F03         Load TR

SWPB R6

MOVB R6,@>5F05         Load SR

SWPB R6

JMP  MOP

*------------------------*

* Track read routine     *

*                        *

* BL   @MTRKR            *

*                        *

* Expects drive selected *

*------------------------*

MTRKR  MOVB @RT,@COMM

LI   R0,READ1

JMP  MTRK1

*------------------------*

* Format track routine   *

*                        *

* BL   @MTRKW            *

* JMP  ERR               *

*                        *

* Expects drive selected *

*------------------------*

MTRKW  MOVB @WT,@COMM         Set command byte to write track

LI   R0,FORMT1         Write track code address

MTRK1  LI   R3,TRKLEN         Length of track

MOP    LI   R1,OVERL1         Move appropriate overlay code into PAD

LI   R2,8

MOP1   MOV  *R0+,*R1+

DEC  R2

JNE  MOP1

BLWP @COMMON           Branch to read/write routine

JMP  MOP5              Command load error

MOP4   A    R3,R4             Update buffer pointer

INCT R11               Increment over error jump

MOP5   RT                     Return

*------------------------------------------------------*

* Head control subroutines with automatic error branch *

*                                                      *

* BL   @TRK00  - Restore head to track zero            *

* BL   @STEPIN - Step head in one track                *

* BL   @SEEK   - Seek track                            *

*                MSB R0 = desired track number         *

*                                                      *

* JMP  ERR     <-----seek/drive  error                 *

*------------------------------------------------------*

TRK00  MOVB @T0,@>5F01        Send RESTORE command to FDC

JMP  INTRQ

STEPIN MOVB @SI,@>5F01        Send STEP-IN command to FDC

JMP  INTRQ

SEEK   MOVB R0,@>5F07         Load DR with desired track in MSB of R0

MOVB @ST,@>5F01        Send SEEK command to FDC

INTRQ  LI   R1,2              Set outside loop counter

INTRQ1 SETO R0                Set error counter

INTRQ2 TB   0                 INTRQ ?

JEQ  INTRQ3            Yes. Exit normally

DEC  R0                Inner loop run out ?

JNE  INTRQ2            No. Keep waiting for INTRQ

DEC  R1                Yes. Outer loop run out ?

JNE  INTRQ1            No. Reset inner loop and wait for INTRQ

JMP  INTRQ4            Yes. Exit with error

INTRQ3 INCT R11               Increment over error jump

INTRQ4 RT                     Return

*-------------------------------------------------------------*

* Common pre-read/write setup routine. Does not reside in PAD *

*                                                             *

* BLWP @COMMON                                                *

* JMP  ERR     <--command load error                          *

*-------------------------------------------------------------*

COMMON DATA GPLWS,COMMN1

COMMN1 MOV  @6(R13),R1

MOV  @8(R13),R4

LI   R2,>4E4E

LI   R12,>1100

SBO  3

SBZ  2

ABS  @SIDE

JEQ  COMMN2

SBO  2

COMMN2 LI   R5,>5F07

BL   @PAD

JMP  COMMN3

MOVB @>5F01,*R13

INCT R14

COMMN3 RTWP

*--------------------------------------------------------------*

* Common read/write code. Installed in PAD by SETUP subroutine *

*                                                              *

* Accessed by BL @PAD (>8300)                                  *

*                                                              *

* Contains 16 byte hole for read/write/format overlay code     *

*--------------------------------------------------------------*

OVERL1 EQU  >831A             Start PAD address of overlay read/write hole

RWCD1  MOVB @COMM,@>5F01      Send read/write command to FDC

SETO R0                Set error counter

RWCD2  TB   1                 DRQ ?

JEQ  RWCD3             Yes. Begin reading/writing

DEC  R0                Error counter run out ?

JEQ  RWCD4             Yes. Error

TB   1                 DRQ ?

JEQ  RWCD3             Yes

TB   0                 INTRQ ?

JNE  RWCD2             No. Keep waiting for DRQ

JMP  RWCD4             Yes. Return to error jump

DATA 0,0               Hole for read/write code

RWCD3  DATA 0,0,0,0,0,0

INCT R11

RWCD4  RT                     Return

RWCD5  EQU  $

RWLGTH EQU  RWCD5-RWCD1       Length of this subroutine

*---------------------------------------------------------------*

* Read and write overlays to be loaded into previous subroutine *

*                                                               *

* READ1      - Read from disk overlay code. All reads supported *

*              Installed in PAD code by BL @SETRD               *

* WRITE1     - Write to disk overlay code. All writes supported *

*              Installed in PAD code by BL @SETWT               *

* FORMT1     - Track write code with track filler loop included *

*---------------------------------------------------------------*

READ1  TB   1                 DRQ ?

JNE  READ2             No

MOVB *R5,*R4+          Yes. Read a byte from the DR

READ2  TB   0                 Now wait for INTRQ

JNE  READ1

NOP                    Space for format code overlay

NOP

NOP

WRITE1 TB   1                 DRQ ?

JNE  WRITE1            No

MOVB *R4+,*R5          Yes. Write a byte to the DR

DEC  R1

JNE  WRITE1            No

WRITE2 TB   0

JNE  WRITE2

NOP

FORMT1 TB   1                 DRQ ?

JNE  FORMT1            No

MOVB *R4+,*R5          Yes. Write a byte to the DR

DEC  R1                Was that the last byte ?

JNE  FORMT1            No

FORMT2 MOVB R2,*R5            Keep sending track filler byte until INTRQ

TB   0

JNE  FORMT2

*--------------------------------------------*

* Routine to fetch FDC status into MSB of R0 *

*                                            *

* BL   @CHKWP                                *

* JMP  WP      <----write protected          *

*--------------------------------------------*

CHKWP  MOVB @>5F01,R0

SLA  R0,2

JOC  CHKWP1

INCT R11

CHKWP1 RT

*----------------------------------------------*

* Turn on DSR rom and appropriate bits routine *

*                                              *

* BL   @ROMON                                  *

*----------------------------------------------*

ROMON  LI   R12,>1100

LDCR @NULL,8

SBO  0

SBO  1

RT