*************************************************************************

*                                                                       *

*   T I  /  M S - D O S   D I S K   M A N A G E M E N T   S Y S T E M   *

*                                                                       *

*                              Version 1.0                              *

*                                                                       *

*-----------------------------------------------------------------------*

*                                                                       *

*                 Designed and written by Will McGovern                 *

*                                                                       *

*                    Copyright 1987 by Will McGovern                    *

*                                                                       *

*                        - ALL RIGHTS RESERVED -                        *

*                                                                       *

*-----------------------------------------------------------------------*

*                                                                       *

*             System requirements :  Myarc disk controller              *

*                                    Two floppy disk drives             *

*                                    32k Expansion memory               *

*                                                                       *

*-----------------------------------------------------------------------*

*                                                                       *

*                           MODULE : MYARC/S                            *

*                                                                       *

*************************************************************************

*================================================================*

*                                                                *

* LEVEL ONE DIRECT ACCESS ROUTINES FOR THE MYARC DISK CONTROLLER *

*                                                                *

*================================================================*

*----------------------------------*

* Data for disk access subroutines *

*----------------------------------*

T0     BYTE >0A               Restore to track zero command

ST     BYTE >1A               Seek track command

SI     BYTE >5A               Step head in command

RS     BYTE >88               Read sector command

WS     BYTE >A8               Write sector command

WT     BYTE >F0               Write track command

COMM   BYTE >00               Command byte

EVEN

*----------------------------------------------------------*

* Subroutines for moving disk read/write code into the PAD *

*                                                          *

* BL @SETUP  - Loads common read write code into the PAD   *

* BL @SETFMT - Loads format code into PAD template         *

* BL @SETRD  - Set PAD routine to read from disk           *

* BL @SETWT  - Set PAD routine to write to disk            *

*----------------------------------------------------------*

SETUP  LI   R0,RWCD1          Set regs to load PAD with read/write code

LI   R1,PAD

LI   R2,RWLGTH

JMP  LDPAD2

SETFMT MOVB @WT,@COMM

LI   R0,FORMT1         Set regs to load in format overlay

JMP  LDPAD1

SETRD  MOVB @RS,@COMM

LI   R0,READ1          Set regs to load in read overlay

JMP  LDPAD1

SETWT  MOVB @WS,@COMM

LI   R0,WRITE1         Set regs to load in write overlay

LDPAD1 LI   R1,OVERL1

LI   R2,8              10 words in each overlay

LDPAD2 MOV  *R0+,*R1+         Move selected code into PAD

DEC  R2

JNE  LDPAD2

RT                     Return

*------------------------------------------------------*

* Head control subroutines with automatic error branch *

*                                                      *

* BL @TRK00  - Restore head to track zero              *

* BL @STEP   - Step head in one track                  *

* BL @SEEK   - Seek track                              *

*              MSB R0 = desired track number           *

*                                                      *

*------------------------------------------------------*

TRK00  MOVB @T0,@>5F01        Send RESTORE command to FDC

JMP  INTRQ

STEP   MOVB @SI,@>5F01        Send STEP-IN command to FDC

JMP  INTRQ

SEEK   MOVB R0,@>5F07         Load DR with desired track in MSB of R0

MOVB @ST,@>5F01        Send SEEK command to FDC

INTRQ  LI   R1,2              Set outside loop counter

INTRQ1 SETO R0                Set error counter

INTRQ2 TB   0                 INTRQ ?

JEQ  INTRQ3            Yes. Exit normally

DEC  R0                Inner loop run out ?

JNE  INTRQ2            No. Keep waiting for INTRQ

DEC  R1                Yes. Outer loop run out ?

JNE  INTRQ1            No. Reset inner loop and wait for INTRQ

JMP  INTRQ4            Yes. Exit with error

INTRQ3 INCT R11               Increment over error jump

INTRQ4 RT                     Return

*----------------------------------------*

* Load track and sector register routine *

*                                        *

* BL @LOADTS - MSB R5 = Track number     *

*              LSB R5 = Sector number    *

*----------------------------------------*

LOADTS MOVB R5,@>5F03         Load TR from MSB of R6

SWPB R5

MOVB R5,@>5F05         Load SR from LSB of R6

SWPB R5                Leave in original condition for return

RT                     Return

*-------------------------------------------------------------*

* Common pre-read/write setup routine. Does not reside in PAD *

*                                                             *

* BL @COMMON - R2 -ve = Side 1 read/write                     *

*              R2 +ve = Side 2 read/write                     *

* D  >xxxx   - Number of bytes to read/write                  *

*-------------------------------------------------------------*

COMMON MOV  *R11+,@GPLWS+2    Load # bytes to read/write in GPL R1

MOV  R4,@GPLWS+8       R4 (CPU address for read/write) to GPL R4

MOVB @FILBYT,@GPLWS+4  Track fill byte in GPL R2 (formatting only)

STWP R0                Preserve current WP

MOV  R0,@COMMN2        Load return WP into LWPI's for return

MOV  R0,@COMMN3

SBZ  2                 Select default side 0

MOV  R2,R2             R2 positive ( side 2 ) ?

JLT  COMMN5            No. Select side 1

SBO  2                 Yes. Select side 2

COMMN5 LWPI GPLWS             Load GPLWS for speed

LI   R10,>5F07

LI   R12,>1100         Load disk CRU base

BL   @>8300            Branch to read/write code in PAD fast ram

COMMN1 LWPI 0                 Error return. Reload return WP

COMMN2 EQU  $-2

RT                     Return to error jump

LWPI 0                 Normal return. Reload return WP

COMMN3 EQU  $-2

INCT R11               Increment R11 over error jump

RT                     Return

*--------------------------------------------------------------*

* Common read/write code. Installed in PAD by SETUP subroutine *

*                                                              *

* Accessed by BL @PAD (>8300)                                  *

*                                                              *

* Contains 20 byte hole for read/write/format overlay code     *

*--------------------------------------------------------------*

OVERL1 EQU  >831A             Start PAD address of overlay read/write hole

RWCD1  MOVB @COMM,@>5F01      Send read/write command to FDC

SETO R0                Set error counter

RWCD2  TB   1                 DRQ ?

JEQ  RWCD4             Yes. Begin reading/writing

DEC  R0                Error counter run out ?

JEQ  RWCD3             Yes. Error

TB   1                 DRQ ?

JEQ  RWCD4             Yes

TB   0                 INTRQ ?

JNE  RWCD2             No. Keep waiting for DRQ

RWCD3  RT                     Yes. Return to error jump

RWCD4  EQU  $+4

BSS  16                Hole for read/write overlay code

AI   R11,6             Increment over error code

RT                     Return

RWCD5  EQU  $

RWLGTH EQU  RWCD5-RWCD1       Length of this subroutine

*---------------------------------------------------------------*

* Read and write overlays to be loaded into previous subroutine *

*                                                               *

* READ1      - Read from disk overlay code. All reads supported *

*              Installed in PAD code by BL @SETRD               *

* WRITE1     - Write to disk overlay code. All writes supported *

*              Installed in PAD code by BL @SETWT               *

* FORMT1     - Track write code with track filler loop          *

*              Installed in PAD code by BL @SETFMT              *

*---------------------------------------------------------------*

READ1  TB   1                 DRQ ?

JNE  READ1             No

MOVB *R10,*R4+         Yes. Read a byte from the DR

DEC  R1                Was that the last byte ?

JNE  READ1             No

READ2  TB   0                 Now wait for INTRQ

JNE  READ2

NOP                    Space for format code overlay

WRITE1 TB   1                 DRQ ?

JNE  WRITE1            No

MOVB *R4+,*R10         Yes. Write a byte to the DR

DEC  R1

JNE  WRITE1            No

WRITE2 TB   0

JNE  WRITE2

NOP

FORMT1 TB   1                 DRQ ?

JNE  FORMT1            No

MOVB *R4+,*R10         Yes. Write a byte to the DR

DEC  R1                Was that the last byte ?

JNE  FORMT1            No

FORMT2 MOVB R2,*R10           Keep sending track filler byte until INTRQ

TB   0

JNE  FORMT2

*-----------------------------------------------------------------------*

* Check FDC status register for errors. Returns to variable error jumps *

*                                                                       *

* Checks bits 1 to 6 for errors and returns to corresponding error jump *

*                                                                       *

* BL  @DSTAT  - Accesses routine                                        *

* JMP ERR1    - Return if bit 1 set                                     *

* JMP ERR2    - Return if bit 2 set                                     *

* JMP ERR3    - Return if bit 3 set                                     *

* JMP ERR4    - Return if bit 4 set                                     *

* JMP ERR5    - Return if bit 5 set                                     *

*-----------------------------------------------------------------------*

DSTAT  MOVB @>5F01,R0         Fetch FDC status register

SLA  R0,1              Ignore NOT READY bit

LI   R1,5              Test 6 bits

DSTAT1 SLA  R0,1              Bit on ?

JOC  DSTAT2            Yes. Return to appropriate error jump

INCT R11               No. Increment over jump for last error

DEC  R1

JNE  DSTAT1

DSTAT2 RT                     Return

*--------------------------------------------*

* Routine to fetch FDC status into MSB of R0 *

*                                            *

* BL @GETSTS - Accesses routine              *

*--------------------------------------------*

SIDE1  SBZ  2

SIDE2  SBO  2

DDEN   SBZ  3

GETSTS MOVB @>5F01,R0

RT

ROMON  LI   R12,>1100

SBO  0

SBO  1

RT