*-------------------------------------------------------------------*

* VDP access routines including inverted string multiple byte write *

*                                                                   *

* Use own workspace. Accessed by BLWP @routine                      *

*                                                                   *

* BLWP @routine                                                     *

*-------------------------------------------------------------------*

VDPWA  EQU  >8C02

VDPWD  EQU  >8C00

VDPRD  EQU  >8800

VDPWSP DATA 0,0,SETWDA        VDP routine workspace

VFILL  DATA VDPWSP,VFILL1     VFILL vector

VMBW   DATA VDPWSP,VMBW1      VMBW vector

VMBR   DATA VDPWSP,VMBR1      VMBR vector

DATA VDPWD,VDPWA

DATA 0,SETRDA,0,0,0

*----------------------------------------------*

* Set VDP mode and load inverted character set *

*                                              *

* BLWP @SETVDP                                 *

*----------------------------------------------*

SETVDP DATA SETWSP,SETV1

SETWSP DATA VREGS,VDPWA,16,VREGS+2,>83D4

DATA >A000,>200,VMBR,VMBW

DATA 0,0,0,0,0,0,0

VREGS  DATA >80,>F081,>82,>E83,>104,>86,>F487

MYCHRS BYTE >00,>00,>00,>07,>0F,>18,>18,>18  TL corner

BYTE >00,>00,>00,>E0,>F0,>18,>18,>18  TR corner

BYTE >18,>18,>18,>F0,>E0,>00,>00,>00  BR corner

BYTE >18,>18,>18,>0F,>07,>00,>00,>00  BL corner

BYTE >00,>00,>00,>FF,>FF,>00,>00,>00  Horizontal border

BYTE >18,>18,>18,>18,>18,>18,>18,>18  Vertical border

TL     EQU  >80

TR     EQU  >81

BR     EQU  >82

BL     EQU  >83

HZ     EQU  >84

VT     EQU  >85

SETV1  LIMI 0                 Disable interrupts

SETV2  MOVB *R0+,*R1          Load VDP registers with new values

DEC  R2

JNE  SETV2

MOVB *R3,*R4           Save VDP R1 at >83D4 for SCAN resetting

BLWP *R7               Read first half of pattern table

DATA >800,>A000,>400

SETV3  INV  *R5+              Create inverse video copies

DEC  R6

JNE  SETV3

BLWP *R8               Write inverse chars into 2nd half

DATA >C00,>A000,>400

BLWP *R8               Write my characters into pattern table

DATA >C00,MYCHRS,48    Character >80 onwards

RTWP

*------------------*

* VDP fill routine *

*------------------*

VFILL1 MOV  *R14+,R0          Fetch VDP address

BL   *R2               Set write address

MOV  *R14+,R1          Fecth # bytes

VFILL2 MOVB *R13,*R9          Fill VDP with byte

NOP

DEC  R1

JNE  VFILL2

RTWP                   Return

*-------------------------*

* VDP multiple byte write *

*-------------------------*

VMBW1  MOV  *R14+,R0          Fetch VDP address

BL   *R2               Set write address

MOV  *R14+,R0          Fetch CPU address

MOV  *R14+,R1          Fetch # bytes

VMBW2  MOVB *R0+,*R9          Write from CPU to VDP

DEC  R1

JNE  VMBW2

RTWP                   Return

*------------------------*

* VDP multiple byte read *

*------------------------*

VMBR1  MOV  *R14+,R0          Fetch VDP address

BL   *R12              Set read address

MOV  *R14+,R0          CPU address

MOV  *R14+,R1          # bytes

VMBR2  MOVB @VDPRD,*R0+       Read from VDP to CPU

DEC  R1

JNE  VMBR2

RTWP                   Return

*--------------------------*

* VDP string write routine *

*--------------------------*

VSTRW  DATA VDPWSP,VSTRW1     Vector for routine. Do NOT put in WSP

VSTRW1 MOV  *R14+,R0          Fetch VDP address

BL   *R2               Set VDP write address

MOV  *R14+,R0          Fetch CPU address of string

VSTRW2 MOVB *R0+,R1           Keep moving data until null byte encountered

JEQ  VSTRW3

MOVB R1,*R9

JMP  VSTRW2

VSTRW3 RTWP                   Return

*----------------------------*

* Reverse video routine      *

*                            *

* BLWP @RVSVID               *

* DATA Screen address        *     *

* DATA # of bytes ( max 40 ) *

*----------------------------*

RVSBUF BSS  40                Byte reversal buffer

RVSVID MOV  *R11,@RVSD1       Load screen address into data

MOV  *R11+,@RVSD2

MOV  *R11,@RVSD1+4     Load # of bytes into data

MOV  *R11,@RVSD2+4

MOV  *R11+,R1

BLWP @VMBR             Read bytes to be inverted from screen

RVSD1  DATA 0,RVSBUF,0

LI   R0,RVSBUF         Start of inverting buffer

RVSV2  MOVB *R0,R2            Fetch byte to be inverted

XOR  @H8000,R2         Invert ( + or - >80)

MOVB R2,*R0+           Save back ifto buffer

DEC  R1

JNE  RVSV2             Finish loop

BLWP @VMBW             Write inverted data back to screen

RVSD2  DATA 0,RVSBUF,0

RT                     Return

*------------------------------------*

* Set VDP read/write address routine *

*                                    *

* BL  @SETWDA/SETRDA                 *

*------------------------------------*

SETWDA ORI  R0,>4000          Set bit 1 if write

SETRDA SWPB R0                LSB first

MOVB R0,*R10

SWPB R0                Now MSB

MOVB R0,*R10

RT                     Return

*----------------------------------------*

* Clear screen routines                  *

*                                        *

* BL @CLRSCR - Clear whole screen        *

*----------------------------------------*

CLRSCR LI   R0,>2000          Space character

BLWP @VFILL            VFILL screen with spaces

DATA 0,960

RT                     Return

*-------------------------------*

* Line drawing routine          *

*                               *

* BL   @LINE - Accesses routine *

* DATA >xxxx - Screen address   *

*-------------------------------*

LINE   MOV  *R11+,@LINE1      Fetch start VDP address for line

H8000  EQU  $+2

LI   R0,>8000          Line character

BLWP @VFILL            Write to line

LINE1  DATA 0,40

RT                     Return

* BLWP @WINDOW

* BYTE X1,Y1,dX,dY

WINDOW DATA WINWSP,WIND1

WINWSP DATA 0,0,0,0,0,0,40,WINDLN

DATA WINDD1,VSTRW,>A000,0

DATA 0,0,0,0

WIND1  MOVB *R14+,R1

SRL  R1,8

MOVB *R14+,R2

SRL  R2,8

MOVB *R14+,R4

SRL  R4,8

MOVB *R14+,R5

SRL  R5,8

DECT R5

MPY  R6,R2

A    R1,R3

BL   *R7

BYTE TL,HZ,TR,0

WIND3  BL   *R7

BYTE VT,>20,VT,0

DEC  R5

JNE  WIND3

BL   *R7

BYTE BL,HZ,BR,0

RTWP

WINDLN MOV  R4,R0

DECT R0

MOV  R10,R1

MOVB *R11+,*R1+

WINDL1 MOVB *R11,*R1+

DEC  R0

JNE  WINDL1

INC  R11

MOVB *R11+,*R1+

MOVB R0,*R1

INC  R11

MOV  R3,*R8

A    R6,R3

BLWP *R9

WINDD1 DATA 0,>A000

RT