VSBR   EQU  >2028

UTWSP  EQU  >2038

FAC    EQU  >834A

SCLEN  EQU  >8354

SCNAME EQU  >8356

DATSTK EQU  >8372

SUBSTK EQU  >8373

CRULST EQU  >83D0

SADDR  EQU  >83D2

GPLWS  EQU  >83E0

GRMRA  EQU  >9802

DORG >24F4

*  DSRLNK routine to load files

DSRLNK DATA DLNKWS,DSLNK

GPLLNK DATA UTWSP,GPLK

DSLNK  MOV  *R14+,@DSRTYP Clear status =s bit

SZCB R12,R15      Clear status =s bit

LWPI GPLWS        Use 16 bit GPL Wsp

MOV  @SCNAME,R0   Fetch name-length pointer

BLWP @VSBR

MOVB R1,R7        Make copy of byte as word

SRL  R7,8

SETO R6           Initialize counter

LI   R2,FAC       Point to FAC as buffer

LNK$LP INC  R0           Point to next byte

INC  R6            & update counter.

C    R6,R7        At end ?

JEQ  LNK$LN       If so, check length

BLWP @VSBR

MOVB R1,*R2+      Standard VSBR

CB   R1,@DECIML   Is it a '.'

JNE  LNK$LP       If not, go for another

LNK$LN MOV  R6,R6        Check length found.

JEQ  DLERR        If =0 error!!

CI   R6,>8        Too long ?

JGT  DLERR        Error !!

LI   R1,SCLEN

MOV  R6,*R1+

INC  *R1          Point to position after name

A    R6,*R1

*  Search for peripheral device

LI   R12,>0F00    Initialize CRU search

NOROM  SBZ  0            Turn off DSR ROM

AI   R12,>100     Point to next DSR space

CLR  @CRULST      Clear in case done

CI   R12,>2000    End of range ?

JEQ  DLERR        Issue error

MOV  R12,@CRULST   else save current CRU

SBO  0             & turn it on.

LI   R2,>4000     Check validation byte

CB   *R2,@HAA      against >AA

JNE  NOROM        If not, there is no ROM there

AI   R2,>8        Point to DSR link

JMP  SGO2

SGO    MOV  @SADDR,R2    Retrieve DSR link

SGO2   MOV  *R2,R2       Link to next

JEQ  NOROM        0 is end in this ROM

MOV  R2,@SADDR    Save latest link

INCT R2           Point to DSR entry

MOV  *R2+,R1      Save and point to len byte

MOV  R6,R5        Copy wanted DSR name length to MSB

SWPB R5

CB   R5,*R2       Compare len bytes

JNE  SGO          Go to next if not same

SWPB R5           Word again

LI   R3,FAC       Point to name in FAC

TYPE   EQU  $

INC  R2           First byte of string

CB   *R3+,*R2     Compare byte by byte

JNE  SGO          Next if match fails

DEC  R5           Keep going till end

DLNKWS JNE  TYPE

BL   *R1          Link to DSR subroutine

JMP  SGO          Oh well, try the next

SBZ  0            Successful DSR call, turn off ROM

LWPI DLNKWS       Reload DSRLNK WS

RTWP

DLERR  LWPI DLNKWS       Unsuccessful DSRLNK

SOCB R12,R15      Set status =s bit

RTWP

HAA    BYTE >AA          Validation byte

DECIML TEXT '.'          Switch delimiter

MASK   DATA >2000        Data in R12

DATA >0,>0,>0     R13-15 for BLWP info

H20    EQU  MASK

DSRTYP EQU  SGO-4

*  BLWP @GPLLNK with GPLLNK EQU >24F8

GPLK   EQU  $

MOVB @GRMRA,R0    Fetch GROM address

SWPB R0

MOVB @GRMRA,R0

SWPB R0

AI   R0,-3        Back off to XML

MOVB @SUBSTK,R1   GPL stack pointer

SRL  R1,8          Word offset

MOV  R0,@>8302(R1) Push GROM address

INCT @>8372        Raise stack pointer

LI   R2,>2000     XML address for XB

MOV  *R2,R3       Save contents

LI   R1,GPLRT     Load return

MOV  R1,*R2       Substitute for previous

MOV  *R14+,@>83EC LNK data to R6 of GPLWS

LWPI GPLWS        Load workspace

B    @>0060       Go to GPL

*  Return from GPL

GPLRT  LWPI >2038        Restore workspace

MOV  R3,*R2       Restore XML contents

RTWP              RETURN to caller

*  Actual assembled program

*  125 w = >FA bytes

AORG >E200         For Vn 4.32  now >100 lower

DATA DLNKWS,DSLNK  4 words

DATA UTWSP,GPLK

*  DSRLNK  90 w

DS1    MOV  *R14+,@DSRTYP 14 w

SZCB R12,R15

LWPI GPLWS

MOV  @SCNAME,R0

BLWP @VSBR

MOVB R1,R7

SRL  R7,8

SETO R6

LI   R2,FAC

DS2    INC  R0            10 w

INC  R6

C    R6,R7

JEQ  DS3

BLWP @VSBR

MOVB R1,*R2+

CB   R1,@DECIML

JNE  DS2

DS3    MOV  R6,R6         10 w

JEQ  DS10

CI   R6,>8

JGT  DS10

LI   R1,SCLEN

MOV  R6,*R1+

INC  *R1

A    R6,*R1

*  Search for peripheral

LI   R12,>0F00     21 w

DS4    SBZ  0

AI   R12,>100

CLR  @CRULST

CI   R12,>2000

JEQ  DS10

MOV  R12,@CRULST

SBO  0

LI   R2,>4000

CB   *R2,@HAA

JNE  DS4

DS5    AI   R2,>8

JMP  DS7

DS6    MOV  @SADDR,R2     15 w

DS7    MOV  *R2,R2

JEQ  DS4

MOV  R2,@SADDR

INCT R2

MOV  *R2+,R1

MOV  R6,R5

SWPB R5

CB   R5,*R2

JNE  DS6

SWPB R5

LI   R3,FAC

DS8    EQU  $             15 w

INC  R2

CB   *R3+,*R2

JNE  DS6

DEC  R5

DS9    JNE  DS8

BL   *R1

JMP  DS6

SBZ  0

LWPI DLNKWS

RTWP

DS10   LWPI DLNKWS

SOCB R12,R15

RTWP

DS11   BYTE >AA           5 w

DS12   TEXT '.'

DS13   DATA >2000

DATA >0,>0,>0

*  GPLLNK  31 w

MOVB @GRMRA,R0     8 w

SWPB R0

MOVB @GRMRA,R0

SWPB R0

AI   R0,-3

MOVB @SUBSTK,R1    19 w

SRL  R1,8

MOV  R0,@>8302(R1)

INCT @>8372

LI   R2,>2000

MOV  *R2,R3

LI   R1,GPLRT

MOV  R1,*R2

MOV  *R14+,@>83EC

LWPI >83E0

B    @>0060

*  Return from GPL

GP1    LWPI >2038         4 w

MOV  R3,*R2

RTWP

END