*  D4MDFM/S   Sep / 09 / 90

*----------------------------

* Myarc direct format routine

*----------------------------

* BLWP @MYFMT

* BYTE DRIVE#(1-4),#SIDES(1/2)

* JMP  ERROR

DEF  MYFMT

COPY "DSK5.D4EQUS/S"

MPAD   EQU  >8300

MGPLWS EQU  >83E0

MBUFFR EQU  >2200

MNULL  DATA >0000

FMTDTA BYTE >00,>0C,>F5,>03,>FE,>01

FOUR   EQU  $+1

BYTE >00,>04,>F7,>01

MONE   EQU  $-1

MB4E   BYTE >4E,>18,>00,>0C,>F5,>03,>FB,>01

BYTE >E5,>80,>E5,>80,>F7,>01,>4E,>18

MBAA   BYTE >AA

INTLCE BYTE 0,11,4,15,8,1,12,5,16

BYTE 9,2,13,6,17,10,3,14,7

EVEN

MYFMT  DATA MYFWSP,MYFMT1

MYFMT1 MOVB *R14,*R14

JEQ  BADDRV

CB   *R14,@FOUR

JLE  MYFT1A

BADDRV RTWP

MYFT1A BLWP @VMBWDF

DATA >3600,MPAD,>100

LI   R0,MCODE

LI   R1,MPAD

MYFMT2 MOV  *R0+,*R1+

JNE  MYFMT2

MYFMT4 LI   R2,MBUFFR

LI   R3,18

LI   R0,>28

MYFMT5 MOVB @MB4E,*R2+

DEC  R0

JNE  MYFMT5

MYFMT6 LI   R4,FMTDTA

MYFMT7 MOVB *R4+,R0

CB   @MBAA,R0

JNE  MYFMT8

DEC  R3

JNE  MYFMT6

JMP  MYFM10

MYFMT8 MOVB *R4+,R1

SRL  R1,8

MYFMT9 MOVB R0,*R2+

DEC  R1

JNE  MYFMT9

JMP  MYFMT7

MYFM10 MOVB *R14+,R0

AI   R0,>300

MOVB R0,@MYFMT3+1

MOVB *R14+,R5

SRL  R5,8

CLR  R6

LI   R12,>1100

LDCR @MNULL,8

SBO  0

SBO  1

MYFMT3 SBO  0

SBZ  2

SBZ  3

RSTORE BL   @TRK00

JMP  MSKERR

MYFM11 LI   R3,40

CLR  R4

MYFM12 LI   R0,MBUFFR+>38

LI   R1,18

LI   R2,INTLCE

MYFM13 MOVB R4,*R0+

MOVB R6,*R0+

MOVB *R2+,*R0+

MOVB @MONE,*R0+

AI   R0,>156-4

DEC  R1

JNE  MYFM13

BLWP @WRTTRK

JMP  WTERR2

DEC  R3

JEQ  MYFM14

BL   @STEP

JMP  MSKERR

AI   R4,>100

JMP  MYFM12

MYFM14 AI   R6,>100

SBO  2

DEC  R5

JNE  RSTORE

INCT R14

WTERR2 EQU  $

MSKERR EQU  $

LDCR @MNULL,8

BLWP @VMBRFW

DATA >3600,MPAD,>100

RTWP

WRTTRK DATA MGPLWS,WRTTK1

TRKLEN EQU  >1830

WRTTK1 LI   R4,>5F07

LI   R5,MBUFFR

LI   R6,TRKLEN

LI   R7,>4E4E

LI   R12,>1100

BL   @MPAD

JMP  WTERR1

INCT R14

WTERR1 RTWP

MCODE  SETO R0

MOVB @WT,@>5F01

MCODE1 TB   1

JEQ  MCODE4

DEC  R0

JEQ  MCODE2

TB   1

JEQ  MCODE4

TB   0

JNE  MCODE1

MCODE2 RT

MCODE3 TB   1

JNE  MCODE3

MCODE4 MOVB *R5+,*R4

DEC  R6

JNE  MCODE3

MCODE5 MOVB R7,*R4

TB   0

JNE  MCODE5

INCT R11

RT

DATA 0

T0     BYTE >02               Restore to track zero command

SI     BYTE >52               Step head in command

WT     BYTE >F0               Write track command

EVEN

*------------------------------------------------------*

* Head control subroutines with automatic error branch *

*                                                      *

* BL   @TRK00  - Restore head to track zero            *

* BL   @STEP   - Step head in one track                *

* BL   @SEEK   - Seek track                            *

*                MSB R0 = desired track number         *

*                                                      *

* JMP  ERR     <-----seek/drive  error                 *

*------------------------------------------------------*

TRK00  MOVB @T0,@>5F01        Send RESTORE command to FDC

JMP  INTRQ

STEP   MOVB @SI,@>5F01        Send STEP-IN command to FDC

INTRQ  LI   R1,2              Set outside loop counter

INTRQ1 SETO R0                Set error counter

INTRQ2 TB   0                 INTRQ ?

JEQ  INTRQ3            Yes. Exit normally

DEC  R0                Inner loop run out ?

JNE  INTRQ2            No. Keep waiting for INTRQ

DEC  R1                Yes. Outer loop run out ?

JNE  INTRQ1            No. Reset inner loop and wait for INTRQ

JMP  INTRQ4            Yes. Exit with error

INTRQ3 INCT R11               Increment over error jump

INTRQ4 RT                     RETURN

END