*  QDMDFM/S   Dec / 11 / 90

*----------------------------

* Myarc direct format routine

*----------------------------

*  Call as  BLWP @MYFMT

*           BYTE DRIVE#(1-4)

*           BYTE #SIDES(1/2)

*  Returns  JMP  ERROR

*

*  Make BLWP call known to world

DEF  MYFMT

*  System REFs

REF  VDPRD,VDPWD

*  Main file REFs

REF  SETRDA,SETWDA,BANK0,BANK1

*  Common EQUate file

COPY "DSK5.QDEQUS/S"

*  Local equates

MPAD   EQU  >8300

MGPLWS EQU  >83E0

SBUFFR EQU  >1000             In VDP

MBUFFR EQU  >1100             In VDP

MNULL  EQU  $

FMTDTA BYTE >00,>0C,>F5,>03,>FE,>01

FOUR   EQU  $+1

BYTE >00,>04,>F7,>01

MONE   EQU  $-1

MB4E   BYTE >4E,>18,>00,>0C,>F5,>03,>FB,>01

BYTE >E5,>80,>E5,>80,>F7,>01,>4E,>18

MBAA   BYTE >AA

INTLCE BYTE 0,11,4,15,8,1,12,5,16

BYTE 9,2,13,6,17,10,3,14,7

EVEN

MYFMT  DATA MYFMWS,MYFMT1

MYFMT1 MOVB *R14,*R14         Check drive #

JEQ  BADDRV            0 not allowed

CB   *R14,@FOUR        4 max

JLE  MYFT1A            Acceptable

BADDRV RTWP

MYFT1A BL   @BANK0            Bank for GSR

BLWP @VMBWDF           Save off PAD contents

DATA SBUFFR,MPAD,>100  Use VDP for buffer

*  Mpve fast code to PAD

LI   R0,MCODE          Fast code data

LI   R1,MPAD           PAD

MYFMT2 MOV  *R0+,*R1+         Move it

JNE  MYFMT2

*  Build track data in VDP

*-------------------------

LI   R9,MBUFFR         VDP track buffer

BL   @SETWDA           Set write

LI   R8,VDPWD          Byte saver

LI   R3,18             Repeat count for 18 s/tk

*  Leader

LI   R0,>28

MYFMT5 MOVB @MB4E,*R8

DEC  R0

JNE  MYFMT5

*  Track header

MYFMT6 LI   R4,FMTDTA

MYFMT7 MOVB *R4+,R0

CB   @MBAA,R0

JNE  MYFMT8

DEC  R3

JNE  MYFMT6

JMP  MYFM10

MYFMT8 MOVB *R4+,R1

SRL  R1,8

MYFMT9 MOVB R0,*R8            @VDPWD

DEC  R1

JNE  MYFMT9

JMP  MYFMT7

*  Set drive #

MYFM10 MOVB *R14+,R0          Fetch drive #

AI   R0,>300           Adjust

MOVB R0,@MYFMT3+1      Construct CRU instruction

*  Set # sides

MOVB *R14+,R5          Second data byte

SRL  R5,8              Word adjust

CLR  R6

*  Set drive and restore to Tk 00

LI   R12,>1100         FDC CRU base

MOV  R12,@MGPLWS+R12+R12

LDCR @MNULL,8          Reset CRU

SBO  0                 DSR ROM on

SBO  1

MYFMT3 SBO  0                 Select drive

SBZ  2

SBZ  3

RSTORE BL   @TRK00

JMP  MSKERR

*  Track write

MYFM11 LI   R3,40             40 tracks

CLR  R4                Null data

*  Write sector interlace into data

MYFM12 LI   R9,MBUFFR+>38

LI   R1,18

LI   R2,INTLCE

MYFA12 BL   @SETWDA

MYFM13 MOVB R4,*R8            Track #

MOVB R6,*R8

MOVB *R2+,*R8

MOVB @MONE,*R8

AI   R9,>156

DEC  R1

JNE  MYFA12

BLWP @WRTTRK

JMP  WTERR2

DEC  R3

JEQ  MYFM14

BL   @STEP

JMP  MSKERR

AI   R4,>100

JMP  MYFM12

MYFM14 AI   R6,>100

SBO  2

DEC  R5

JNE  RSTORE

INCT R14

WTERR2 EQU  $

MSKERR EQU  $

LDCR @MNULL,8

BLWP @VMBRFW

DATA SBUFFR,MPAD,>100

BL   @BANK1

RTWP

*  Track write from VDP data

*---------------------------

WRTTRK DATA MGPLWS,WRTTK1

TRKLEN EQU  >1830

WRTTK1 LI   R4,>5F07

LI   R8,VDPRD

LI   R9,MBUFFR

BL   @SETRDA

LI   R6,TRKLEN

LI   R7,>4E4E

BL   @MPAD

RTWP

INCT R14

RTWP

*  Fast code to move to PAD for execution

*----------------------------------------

MCODE  SETO R0

MOVB @WT,@>5F01

MCODE1 TB   1

JEQ  MCODE4

DEC  R0

JEQ  MCODE2

TB   1

JEQ  MCODE4

TB   0

JNE  MCODE1

MCODE2 RT

MCODE3 TB   1

JNE  MCODE3

MCODE4 MOVB *R8,*R4           From VDPRD

DEC  R6

JNE  MCODE3

MCODE5 MOVB R7,*R4

TB   0

JNE  MCODE5

INCT R11

RT

DATA 0

T0     BYTE >02               Restore to track zero command

SI     BYTE >52               Step head in command

WT     BYTE >F0               Write track command

EVEN

*------------------------------------------------------*

* Head control subroutines with automatic error branch *

*                                                      *

* BL   @TRK00  - Restore head to track zero            *

* BL   @STEP   - Step head in one track                *

* BL   @SEEK   - Seek track                            *

*                MSB R0 = desired track number         *

*                                                      *

* JMP  ERR     <-----seek/drive  error                 *

*------------------------------------------------------*

TRK00  MOVB @T0,@>5F01        Send RESTORE command to FDC

JMP  INTRQ

STEP   MOVB @SI,@>5F01        Send STEP-IN command to FDC

INTRQ  LI   R1,2              Set outside loop counter

INTRQ1 SETO R0                Set error counter

INTRQ2 TB   0                 INTRQ ?

JEQ  INTRQ3            Yes. Exit normally

DEC  R0                Inner loop run out ?

JNE  INTRQ2            No. Keep waiting for INTRQ

DEC  R1                Yes. Outer loop run out ?

JNE  INTRQ1            No. Reset inner loop and wait for INTRQ

JMP  INTRQ4            Yes. Exit with error

INTRQ3 INCT R11               Increment over error jump

INTRQ4 RT                     RETURN

END