*  FILESAVE UTILITY FOR FUNNELWEB DR80

IDT  'DRSAVE'

REF  SFIRST,SLAST,VMBW

DEF  SAVE

*  Equates from FWB

DSRLNK EQU  >FFD4        BLWP vectors

VMBWD  EQU  >FFBA

VSBRD  EQU  >FFB2

BTLN   EQU  >FF58

FWREGS EQU  >FF7C

*  FILSAV equates

PAB    EQU  >FD0

PABBUF EQU  >1000

KEYBRD EQU  >8374

GPLST  EQU  >837C

GPLWS  EQU  >83E0

LONG   EQU  >2700-6   >37D6->1006

NEXTHI EQU  >A050+LONG

SHORT  EQU  >1FFA

*  Entry points, continue in USRWSP

AORG >2676

SAVE   EQU  $

LWPI FWREGS

MOV  @BTLN,R1

MOVB @16(R1),@FILENM-3

BLWP *R9               First file program data

DATA PABBUF+6

DATA >A050

DATA LONG

LI   R3,LONG+6         File length incl header

BL   @FILSAV

DATA HEAD1

LI   R2,SLAST          Final pointer in hi-mem

LI   R1,NEXTHI         End of first

S    R1,R2             Length of last

MOV  R1,@HEAD2+4       Write to header

MOV  R2,@HEAD2+2       Length also

LI   R0,PABBUF+6

BLWP @VMBW

MOV  R2,R3

AI   R3,6

BL   @FILSAV

DATA HEAD2

MOV  @>FF5C,R11

B    *R11

**********  SUBROUTINE  ***********

FILSAV EQU  $

MOV  *R11+,@FLSVD

MOV  R3,@FILLEN

AB   @ONE,@FILENM

BLWP *R9

DATA PABBUF

FLSVD  DATA >0,6

BLWP *R9

DATA PAB,PABDT,>20

LI   R0,PAB+9

MOV  R0,@>8356

BLWP @DSRLNK      Call DSR

JEQ  FAIL         Not found

FLSV1  BLWP @VSBRD       Fetch status from PAB

DATA PAB+1

SRL  R0,13        3 bits only

JNE  FAIL         Any set

MOVB @GPLST,R0    Fetch GPL status byte

SLA  R0,3         Test =s bit

JOC  FAIL

RT

FAIL   BLWP @>0

*  DATA and PAB template

PABDT  DATA >0600,PABBUF,>0

FILLEN DATA >0

DATA >07

TEXT 'DSK5.D'

FILENM TEXT 'Q'

ONE    BYTE >01

EVEN

HEAD1  DATA >FFFF,LONG,>A050

HEAD2  DATA >0000,>0,NEXTHI

HEAD3  DATA >0000,>0,>0

*  Byte and text data

ERM3   TEXT 'SAVE Error'

EVEN

END