*  QDAVPC/D  May / 31 / 91

*------------------------------------------------------------

*  PGDIS  Display on screen a 20 line page of catalog entries

*         Line # in R3 - selects page containing that line

*------------------------------------------------------------

*  Subroutine to locate top of page and cursor bar

*-------------------------------------------------

TOPOPG EQU  $

MOV  R3,R15            Copy current file pointer

CLR  R14               Set up MSW for division

DIV  @W20,R14          # of files per page

MOV  R14,R9            Page # for immediate use

MOV  R15,@CSRFIL       Copy # files down on page

MPY  @W20,R14          Calculate file # at top of page

MOV  R15,@PGFTOP       Save for use

MOV  R15,R14           Get ready for next MPY

MPY  @W0E,R14          Offset to file entry in CATBF

AI   R15,CATBF         Set up CATBF first page pointer

RT

*  Main directory page display

*-----------------------------

PGDIS  EQU  $

MOV  R11,@PGD99+2      Save return

BL   @BEEPL

BL   @TOPOPG           Get page details

BL   @PGC              Page # already in R9

CLR  R4                Start counter for 20 files per page

LI   R13,SC1+FLSTT     One line up from screen address at top

PDS1   MOV  R15,R9            Copy catalog buffer pointer

BL   @VMBRE            Fetch from VDP

DATA DRBF+2,>0E        14 bytes into DRBF

BL   @UNPACK           Decompress entry

MOV  R13,R9            Copy pointer for use

BLWP @VSTRWR           Write entry to screen

MOV  R4,R8             Copy counter on page

MPY  @TEN1,R8          Bytes into color table

AI   R9,COLTB1+46      Offset from first

MOV  R9,R0             Save pointer

BL   @VMBWE            Write up color bar

DATA DBARE,4

*  Write up cursor

C    R4,@CSRFIL        Is it the one with the cursor

JNE  PDS2              No

MOV  R13,R9            Screen pointer

BL   @VSTRM            Write in front of entry

MOV  R13,R9            Copy  name screen pointer

AI   R9,31             At end of name

BL   @VSTRM            Write up mark

MOV  R0,R9

BL   @VMBWE

DATA DBARW,4

*  Write up file tag marks

PDS2   MOV  @PGFTOP,R11       Reconstruct current file #

A    R4,R11            Top of page + current file on page

MOVB @TAGFL(R11),R11   Is it tagged ?

JEQ  PDS3              No

MOV  R13,R9            In front of file again

AI   R9,11             Middle marker

BL   @VSTAG            Write Tag mark

*  Continus page display

PDS3   INC  R4                Count one done

AI   R15,>0E           Update buffer pointer

AI   R13,LLL           Update screen pointer

CI   R4,20             Page's worth ?

JL   PDS1              Repeat if more to go

PGD99  B    @>0               Restore return address

*  PGC/PGM  Display page #

*  -----------------------

PGC    EQU  $

LI   R0,SC1+PAGNA      For current page display

JMP  PGN

PGM    LI   R0,SC1+PAGNB      For max page display

PGN    MOV  R11,R14           Save return

INC  R9                Make into actual page count

MOV  R9,R8             Copy page #

SWPB R8                For byte op

AI   R8,>3000          Add ASCII bias

MOV  R0,R9             Fetch screen pointer

BL   @SETWDA           Set for write

MOVB R8,@VDPWD         Only 1 digit

B    *R14

*  Buffer storage format for each file record

*--------------------------------------------

*

*  Compressed to 14 bytes per entry  --->>

*

*  < 10 chr filename > < size word > < reclen byte > < info byte >

*

*   <info byte>  --->>

*

*   <2 bits program type, 1 bit fractures, 1 bit protection,

*      1 unused bit, 3 bits file type>

*

*  Unpack Buffer entry

*  -------------------

UNPACK EQU  $

MOV  R11,R10           Save return

LI   R11,>2020         Set length byte

H1E    EQU  $-2               Handy data

LI   R14,DRBF          Point to CRAM display buffer

MOV  R11,*R14+         At start of buffer area

LI   R6,32             Display buffer length

C    *R14,R11          Spaces if no entry

JNE  UNPK05            Entry exists

BL   @MFILL            Fill with spaces

JMP  UNPK40            Return immediately

UNPK05 SRL  R6,1              Half of buffer

LI   R14,DRBF+16       Starting from the middle

BL   @MFILL            Fill with spaces

CLR  R8                Clear LSB

MOVB @DRBF+15,R8       Frac+Prot+Type encoded byte

MOV  R8,R9             Copy for use

ANDI R9,>C000          Isolate 2 MSbits for Pgm-type if set

SRL  R9,13                Make word index x 2

MOVB @PRGTAB(R9),@DRBF+27   Get display bytes

MOVB @PRGTAB+1(R9),@DRBF+28  Odd/even unfortunately

SLA  R8,3                   Fractured file ?

JNC  UNPK10            No - leave it blank

MOVB @STAR,@DRBF+17    Write "*" to indicate

UNPK10 SLA  R8,1              Protected file ?

JNC  UNPK20            No - leave it blank

MOVB @ASCP,@DRBF+30    Write "P" in last column

UNPK20 SRL  R8,9              Isolate File-type index x 8

AI   R8,TYPTBL         Index into name list

LI   R0,DRBF+18        Buffer pointer for display

MOV  *R8+,*R0+         Move 8 bytes

MOV  *R8+,*R0+

MOV  *R8+,*R0+

MOV  *R8,*R0+          Leave R0 pointing to next entry

MOVB @DRBF+14,R9       Record length byte

JEQ  UNPK30            Must be program file

SRL  R9,8              Word value

BL   @RJNUM            Convert to string

LI   R6,RJF#+1         Write to buffer

MOVB *R6+,*R0+         Byte address unfortunately

MOVB *R6+,*R0+

MOVB *R6,*R0

UNPK30 MOV  @DRBF+12,R9       Fetch size word for body of file

MOVB @H20,@DRBF+12     Clear where it was

MOV  @SPERAU,R11       Fetch sectors/AU

A    R11,R9            Allow for FDR AU

DEC  R11               Round up for display purposes

A    R11,R9            Add 1 if 2sec/AU

SZC  R11,R9            Cancel if already even

BL   @RJNUM            Convert to decimal

LI   R6,4                    Could be 4 digits

UNPK32 MOVB @RJF#-1(R6),@DRBF+12(R6)  Move to display buffer

DEC  R6                      Tick off

JGT  UNPK32             Loop on it

UNPK40 LI   R8,DRBF            Preload expected in several places

B    *R10

*  DSR Sub-program routines

*  Direct entry for SECReaD, SECWRite

*  Others as DIRect_ACcess

*------------------------------------

*  Call as  LI   R7,<sector #>

*           BL   @SECRD/WR

*           DATA <error return>

*

*       or  LI   R14,<PAB data in CPU>

*           BL   @DIRAC

*           DATA <error return>

*     with  <file name data> at MFBUF

*

*  Null <error> return flags bypass of internal error screen

*  and returns to address at ONNNBG for special handling.

*

*  Uses  R8 -- R12, R14

*  Exits in VBank #1

DIRNBG DATA DRIVEQ

OPNNBG DATA >0

SECWR  EQU  $

MOVB @NULL,@DR#+1      Set for write

JMP  SECRE

SECRD  EQU  $

MOVB @HFF,@DR#+1       Make sure it is Read

SECRE  MOV  R7,@SECT#         Load sector #

LI   R14,SEC           Sector data

MOV  R11,R10           Save return

BL   @BANK0            Set Vbank #0 for disk ops

JMP  DIRAC1

DIRAC  EQU  $

MOV  R11,R10           Save return

BL   @RESCRU           Restart CRU search

BL   @BANK0            Set Vbank #0 for disk ops

BLWP @VMBWDF           Load up filenames

DATA AVPAC,MFBUF,20

DIRAC1 MOV  *R10+,@DIRNBG     Collect error exit

MOV  @DSRLNK+2,R11     Fetch routine pointer

INCT @DSRTYP(R11)      Alter access type

DIRRN  MOV  R14,R12

MOV  *R12+,R8

LI   R9,AVPAB          Point to PAB

MOV  R9,@SCNAME        Write for DSR

BLWP @VSTRWR           Write to VDP

LI   R11,FAC+2         Byte saver

MOV  *R12+,*R11+        Disk drive / opcode

MOV  *R12+,*R11+       Set for read op

MOV  *R12,*R11         Point to sector buffer

BLWP @DSRLNK           Find & execute DSR

JEQ  DIERR             Fatal error

MOVB *R11,R12          Check if error returned

JEQ  DIRREX            All seems OK

DIERT  MOV  @DSRLNK+2,R11     Routine pointer

AB   @ONE,@CRUST(R11)  Update CRU start

CB   @CRUST(R11),@H1F  At end ?

JL   DIRRN             Not yet

DIERR  MOV  @DIRNBG,R10       Bypass error warnings on screen ?

JNE  DIRREU            No - use regular path

MOV  @OPNNBG,R10       Null flags special error handler

BL   @RESCRU           Reset CRU base

JMP  DIRREX

*  Normal error handler

DIRREU BL   @BANK1            Restore QD bank for message

BL   @INCLR            Clear box

BL   @VSTRU            Write return msg

DATA ERMSH,MSLN4+8

BL   @SHOERP           Write to screen

BL   @INCLR            Clear box

BL   @RESCRU           Reset CRU

MOV  @DIRNBG,R10       Force exit branch

*  Normal exit

DIRREX MOV  @DSRLNK+2,R9      Fetch pointer to routine

DECT @DSRTYP(R9)       Restore file type access

BL   @BANK1            Restore QD bank for normal exit

B    *R10              Exit with R9 pointing to DSRLNK+2

RESCRU MOV  @DSRLNK+2,R9      Fetch pointer to routine

MOVB @H0F,@CRUST(R9)

RT

*  String write to VDP

*---------------------

VSTAG  LI   R8,TAGMK          For tagging files

JMP  VSTRW

VSTRM  LI   R8,MARKR          For writing up cursor marks

JMP  VSTRW

VSTRU  MOV  *R11+,R8          All in-line data entry

VSTRV  MOV  *R11+,R9          Partial - string pointer in R8

VSTRW  BLWP @VSTRWR           All register entry

RT

*  VMBWD as a BL routine

*-----------------------

VMBWD  MOV  *R11+,R9          All in-line data entry

VMBWE  MOV  *R11+,R8          CRAM pointer

PLACE  MOV  R11,R14           Save return

BL   @SETWDA           Set VDP write

MOV  *R14+,R9          Now use R9 to hold count data

VST1   MOVB *R8+,@VDPWD       Write to VRAM

DEC  R9                Count off

JGT  VST1              More to do

B    *R14              Return

*  VMBRD variations as BL routines

*---------------------------------

VMBRD  EQU  $                 All in-line data entry

MOV  *R11+,R9          VRAM pointer

VMBRE  MOV  *R11+,R8          CRAM pointer

FETCH  MOV  *R11+,R6          # bytes to write

FETCH2 MOV  R11,R14           Save return

BL   @SETRDA           Set VDP read

VMBR1  MOVB @VDPRD,*R8+       Fetch a byte

DEC  R6                Count it

JGT  VMBR1             More to do

B    *R14              Return

*  VFILL   VDP fill from MSB of R8

*          Count in R6, VDPA in R9, uses R14

*  -----------------------------------------

VFILI  LI   R8,>2000          Preload blank char

VFILJ  MOV  *R11+,R6          All in-line data entry

VFILK  MOV  *R11+,R9          Entry with count in R6

BLWP @VFILRR           Call UTIL routine

RT

*  VDP Address setting

*---------------------

SETWDA EQU  $

ORI  R9,>4000          Set write flag bit

SETRDA SWPB R9                LSByte first

MOVB R9,@VDPWA         Byte to VDP

SWPB R9                Swap and delay

MOVB R9,@VDPWA         MSbyte to VDP

ANDI R9,>3FFF          Reset any flag bits

RT

*  Switch VDP banks for R/W

*--------------------------

BANK0  EQU  $

MOVB @NULL,@VDPWA      Normal TI bank

JMP  BANKT             Does for delay too

VRAM   EQU  $

MOVB @NULL,@VDPWA      Reset expansion RAM access bit

NOP                    WASTE time

MOVB @HAD,@VDPWA       VR #45

NOP

BANK1  EQU  $

MOVB @ONE,@VDPWA       First AVPC bank

JMP  BANKT

BANKE  EQU  $

MOVB @H40,@VDPWA       Set expansion RAM access bit

NOP                    DELAY

MOVB @HAD,@VDPWA       Assign to VR#45

JMP  BANKVE

BANKV  EQU  $

AB   @FOUR,R0          Expects offset from Vbank #4 in R0

BANKVE MOVB R0,@VDPWA         Write offset value

NOP                    DELAY

BANKT  MOVB @H8E,@VDPWA       Specify bank switching register as target

RT

*  Fill memory block

*-------------------

MINIT  LI   R14,LINBUF        Assume standard line buffer

LI   R6,>100           Buffer length for max record

MFILL  MOVB @SPACE,*R14+      Set to ASCII blank

DEC  R6                One done

JGT  MFILL             Back for more

RT                     RETURN to caller