|  |  |  |  |  |  |  |  |  |  |  | ● ● |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DEL | INS | ERASE | CLEAR | BEGIN | PROC'D | AID | REDO | BACK |  | QUIT |  |
| ! 1 | @ 2 | # 3 | $ 4 | % 5 | ^ 6 | & 7 | * 8 | ( 9 | ) 0 | + = |  |
| 8→7 | 13→7 | 14→7 | 15→7 | 9→7 | 9→2 | 15→2 | 14→2 | 13→2 | 8→2 | 12→5 |  |
| 001, -INT7 | 101, -INT7 | 110, -INT7 | 111, -INT7 | 000, -INT7 | 000, -INT6 | 111, -INT6 | 110, -INT6 | 101, -INT6 | 001, -INT6 | 100, -INT3 |  |

| Q | W ~ | E ↑ | R [ | T ] | Y | U _ | I ? | O ' | P " | - / |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8→10 | 13→10 | 14→10 | 15→10 | 9→10 | 9→1 | 15→1 | 14→1 | 13→1 | 8→1 | 8→5 |
| 001, -INT9 | 101, -INT9 | 110, -INT9 | 111, -INT9 | 000, -INT9 | 000, -INT5 | 111, -INT5 | 110, -INT5 | 101, -INT5 | 001, -INT5 | 001, -INT3 |

| A ¦ | S ← | D → | F { | G } | H | J | K | L | : ; | ENTER ● |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8→3 | 13→3 | 14→3 | 15→3 | 9→3 | 9→4 | 15→4 | 14→4 | 13→4 | 8→4 | 12→1 |
| 001, -INT8 | 101, -INT8 | 110, -INT8 | 111, -INT8 | 000, -INT8 | 000, -INT4 | 111, -INT4 | 110, -INT4 | 101, -INT4 | 001, -INT4 | 100, -INT5 |

| SHIFT | Z \ | X ↓ | C ` | V | B | N | M | < , | > . | SHIFT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12→3 | 8→11 | 13→11 | 14→11 | 15→11 | 9→11 | 9→5 | 15→5 | 14→5 | 13→5 | 12→3 |
| 100, -INT8 | 001, -INT10 | 101, -INT10 | 110, -INT10 | 111, -INT10 | 000, -INT10 | 000, -INT3 | 111, -INT3 | 110, -INT3 | 101, -INT3 | 100, -INT8 |

| ALPHA LOCK | CTRL ● |  |  |  |  |  |  |  |  | FCTN ● |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6→7 | 12→10 | 12→4 |  |  |  |  |  |  |  | 12→7 |
| P5,-INT7 | 100, -INT9 | 100, -INT4 |  |  |  |  |  |  |  | 100, -INT7 |

The first pair of numbers below each key represents the pins that closing the key's switch will short - the number on the left is the output from the CRU [on the keyboard connector], and the number on the right is the input interrupt line [also on the keyboard connector].  If you're looking at the keyboard from the top, the pins are numbered left-to-right.

The second pair of numbers consists of a binary combination and an interrupt.  This represents the combination of output bits P2, P3 and P4, and the interrupt line that will be triggered if the key is pressed at the same time.  For example, for <SPACE>, "100,-INT4" means that P2 should be HIGH, while P3 and P4 are LOW, and, if depressed, interrupt line -INT4 will be triggered.  Of note is the exception, ALPHA LOCK.  This key doesn’t use P2, P3 orP4, but triggers an interrupt on -INT7 any time P5 is HIGH and the key is depressed.

- Brian Tristam Williams